# Arquitectura de Computadores

# Conjunto de Instruções da família de P Intel 80x86

Versão 1.0

# (Retirado de Norton Guide)

# Departamento de Engenharia Informática

# ISEP - IPP

# Junho de 1996

# OVERVIEW

# Overview of the Instruction Format

### See Also: EA, Flags

Each entry in this list includes information on which flags in the 8088's flag register are changed, and how they're changed.  Since there are 9 flags in the flags register, the flags display is very compact:

| Flags: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 |  | * | * | ? | * | 0 |

? Undefined after the operation.

* Changed to reflect the results of the instruction.

0 Always cleared

1 Always set

The timing charts show timings for the 8088.  Since the 80x8x processors execute instructions in fewer clock cycles than the 8088, these charts represent the worst case.

Operands	This field gives the list of possible operands and addressing modes for each instruction.

Clocks	Number of clock cycles required to execute the instruction on an 8088.  Effective Address calculations (EA) take additional time, as outline in the EA table.

Transfers	The number of memory references.  4 clock cycles are required for each memory reference.

Bytes	Number of bytes in the instruction.

Note: The additional clock cycles required to reinitialize the instruction que and fetch the next instruction after a control transfer instruction (such as JMP or CALL) is already included in the timing tables. Two clock times are listed for conditional transfer instructions (such as JZ); the shortest time is for the case when there is no transfer.

Flags Register

The flags register contains various bits that control and record the state of the microprocessor, as defined below.

| Bit: | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Flags Register: |  |  |  |  | OF | DF | IF | TF | SF | ZF |  | AF |  | PF |  | CF |

Overflow	Set when an arithmetic overflow occurred.  An arithmetic overflow occurs when the size of a computation exceeds the size of the destination.

Direction	Set for auto-decrement with string instructions, clear for auto-increment.

Interrupt Enable	Interrupts are enabled as long as this flag is set.  When this flag is cleared, interrupts except for nonmaskable interrupts are disabled.

Trap	This flag is used by debuggers to single step through programs. When this flag is set, an INT 3 is generated after every instruction.

Sign	Set when the high-order bit of the result is 1. In other words, S = 0 for positive numbers and S = 1 for negative numbers.

ZeroSet	Whenever the result is 0.

Auxiliary Carry	Set when the is a carry out of the lower half of an 8 or 16 bit number, or when there is a borrow from the upper to the lower half.This flag is used mainly by the decimal-arithmetic instructions.

Parity Flag	Set if there is an even number of 1-bits in the result.Cleared if there is an odd number of 1-bits.Often used by communications programs

Carry Flag	Set if there was a carry out of, or a borrow into the high-order bit of the result.This flag is useful for propagating carries and borrows for multi-word numbers.

# Effective Address (EA) Calculations

This table lists the number of clock cycles required to calculate the effective address on the 8088 microprocessor.The 80x8x processors require considerably fewer clock cycles to calculate effective addresses, so this table represents the worst-case.

| EA Component | 8088 Clocks | Example |
| --- | --- | --- |
| Displacement | 6 | MOV AX,ADDR |
| Register indirect BX, BP, SI, DI | 5 | MOV AX,[BP] |
| Displacement + Base or Index BX + Disp, BP + Disp SI + Disp, DI + Disp | 9 | MOV AX,ADDR[BP] |
| Base + Index BP + DI, BX + SI | 7 | MOV AX,[BP+DI] |
| BP + SI, BX + DI | 8 | MOV AX,[BX+DI] |
| Displacement + Base + Index |  |  |
| BP + DI + Disp | 11 | MOV AX,ADDR[BP+DI] |
| BX + SI + Disp |  |  |
| BP + SI + Disp | 12 | MOV AX,ADDR[BP+SI] |
| BX + DI + Disp |  |  |

Note: Add 2 clocks for segment overrides.

Each memory reference requires an additional 4 clock cycles.The Transfers field in the instruction timing charts gives the number of memory references for each instruction.

# Instruction Set

# AAA	ASCII Adjust after Addition

### See also: AAD, AAS, AAM, ADC, DAA, Flags

| Flags Affected: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | * |  |  |  | * | * | ? | * | ? |

## AAA

Logic:	If (AL & 0Fh) > 9 or (AF = 1) then

AL  AL + 6

AH AH + 1

AF 1;CF  1

else

AF  0;CF  0

AL  AL & 0Fh

Converts the number in the lower 4 bits (nibble) of AL to an unpacked BCD number (high-order nibble of AL is zeroed).

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| no operands | 4 | - | 1 | AAA |

If the lower 4 bits of the number in AL is greater than 9, or the auxiliary carry flag is set, this instruction converts AL to its unpacked BCD form by adding 6 (subtracting 10) to AL; adding 1 to AH; and setting the auxiliary flag and carry flags. This instruction will always leave 0 in the upper nibble of AL.

Note: Unpacked BCD stores one digit per byte; AH contains the most-significant digit and AL the least-significant digit.

# AAD	ASCII Adjust before Division

### See Also: AAA, AAS, AAM, DIV, IDIV, Flags

| Flags Affected: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ? |  |  |  | * | * | ? | * | ? |

## AAD

Logic:	AL  AH * 10 + AL

AH  0

AAD converts the unpacked two-digit BCD number in AX into binary in preparation for a division using DIV or IDIV, which require binary rather than BCD numbers.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| no operands | 60 | - | 2 | AAD |

AAD modifies the numerator in AL so that the result produced by a division will be a valid unpacked BCD number. For the subsequent DIV to produce the correct result, AH must be 0. After the division, the quotient is returned in AL, and the remainder in AH. Both high-order nibbles are zeroed.

Note: Unpacked BCD stores one digit per byte; AH contains the most-significant digit and AL the least-significant digit.

# AAM	ASCII Adjust after Multiply

### See also: AAA, AAD, AAS, MUL, IMUL, Flags

| Flags Affected: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ? |  |  |  | * | * | ? | * | ? |

## AAM

Logic:	AH  AL / 10

AL  AL MOD 10

This instruction corrects the result of a previous multiplication of two valid unpacked BCD operands. A valid 2-digit unpacked BCD number is taken from AX, the adjustment is performed, and the result is returned to AX. The high-order nibbles of the operands that were multiplied must have been 0 for this instruction to produce a correct result.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| no operands | 83 | - | 1 | AAM |

Note: Unpacked BCD stores one digit per byte; AH contains the most-significant digit and AL the least-significant digit.

# AAS	ASCII Adjust after Subtraction

### See Also: AAA, AAD, AAS, SUB, SBB, DAS, Flags

| Flags Affected: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | * |  |  |  | * | * | ? | * | ? |

## AAS

Logic:			If (AL & 0Fh) > 9 or AF = 1 then

AL  AL - 6

AH  AH - 1

AF  1;CF  1

else

AF  0;CF  0

AL  AL & 0Fh

AAS corrects the result of a previous subtraction of two valid unpacked BCD operands, changing the content of AL to a valid BCD number. The destination operand of the subtraction must have been specified as AL. The high-order nibble of AL is always set to 0.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| no operands | 4 | - | 1 | AAS |

Note: Unpacked BCD stores one digit per byte; AH contains the most-significant digit and AL the least-significant digit.

# ADC 	Add with Carry

### See Also: ADD, INC, AAA, DAA, EA, Flags

| Flags Affected: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | * |  |  |  | * | * | * | * | * |

## ADC destination,source

Logic:			destination  destination + source + CF

ADC adds the operands, adds 1 if the Carry Flag is set, and places the resulting sum in destination. Both operands may be bytes or words, and both may be signed or unsigned binary numbers.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
|  | byte(word) |  |  |  |
| register, register | 3 | - | 2 | ADC BX,SI |
| register, immediate | 4 | - | 3-4 | ADC CX,128 |
| accumulator, immediate | 4 | - | 2-3 | ADC AL,10 |
| register, memory | 9(13)+EA | 1 | 2-4 | ADC DX,RESULT |
| memory, register | 16(24)+EA | 2 | 2-4 | ADC BETA,DI |
| memory, immediate | 17(25)+EA | 2 | 3-6 | ADC GAMMA,16h |

Note: ADC is useful for adding numbers that are larger than 16 bits, since it adds a carry from a previous operation.

# ADD	Addition

### See Also: ADC, INC, AAA, DAA, EA, Flags

| Flags Affected: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | * |  |  |  | * | * | * | * | * |

## ADD destination,source

Logic:			destination  destination + source

ADD sums the operands and stores the result in destination. Both operands may be bytes or words, and both may be signed or unsigned binary numbers.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
|  | byte (word) |  |  |  |
| register, register | 3 | - | 2 | ADD BX,CX |
| accumulator, immediate | 4 | - | 2-3 | ADD AX,256 |
| register, immediate | 4 | - | 3-4 | ADD BL,4 |
| register, memory | 9(13)+EA | 1 | 2-4 | ADD DI,[DX] |
| memory, register | 16(24)+EA | 2 | 2-4 | ADD TOTAL,BL |
| memory, immediate | 17(25)+EA | 2 | 3-6 | ADD RESULT,3 |

# AND	Logical AND

### See also: NOT, OR, XOR, EA, Flags

| Flags Affected: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 |  |  |  | * | * | ? | * | 0 |

## AND destination,source

Logic:			destination  destination AND source

AND performs a bit-by-bit logical AND operation on its operands and stores the result in destination. The operands may be words or bytes.

AND Instruction Logic:

| Destination | Source | Result |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 0 |
| 1 | 0 | 0 |
| 1 | 1 | 1 |

AND sets each bit of the result to 1 if both of the corresponding bits of the operands are 1.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
|  | byte(word) |  |  |  |
| register, register | 3 | - | 2 | AND AL,DL |
| register, immediate | 4 | - | 3-4 | AND CX,0FFh |
| accumulator, immediate | 4 | - | 2-3 | AND AX,01000010b |
| register, memory | 9(13)+EA | 1 | 2-4 | AND CX,MASK |
| memory, register | 16(24)+EA | 2 | 2-4 | AND VALUE,BL |
| memory, immediate | 17(25)+EA | 2 | 3-6 | AND STATUS,01h |

# CALL	Call Procedure

### See also: RET, JMP, PROC, NEAR, FAR, EA

Flags not altered

## CALL procedure_name

Logic:		if FAR CALL (inter-segment)

PUSH CS

CS  dest_seg

PUSH IP

IP  dest_offset

CALL transfers control to a procedure that can either be within the current segment (a NEAR procedure) or outside it (a FAR procedure). The two types of CALLs result in different machine instructions, and the RET instruction that exits from the procedure must match the type of the CALL instruction (the potential for mismatch exists if the procedure and the CALL are assembled separately).

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
|  | byte(word) |  |  |  |
| near-proc | 19(23) | 1 | 3 | CALL NEAR_PROC |
| far-proc | 28(36) | 2 | 5 | CALL FAR_PROC |
| memptr 16 | 21(29)+EA | 2 | 2-4 | CALL PROC_TABLE[SI] |
| regptr 16 | 16(24) | 1 | 2 | CALL AX |
| memptr 32 | 37(57)+EA | 4 | 2-4 | CALL [BX].ROUTINE |

Note: For an inter-segment procedure (procedure in a different segment), the processor first pushes the current value of CS onto the stack, then pushes the current value of IP (which is pointing to the instruction following the CALL instruction), then transfers control to the procedure.

For an intra-segment procedure (procedure in the same segment), the processor first pushes the current value of IP (which is pointing to the instruction following the CALL instruction) onto the stack, then transfers control to the procedure.

CALL can also read the procedure address from a register or memory location. This form of CALL is called an indirect CALL.

# CBW	Convert Byte to Word

### See also: CWD, DIV, IDIV

Flags not altered

## CBW

Logic:		if (AL < 80h) then

AH  0

else

AH  FFh

CBW extends the sign bit of the AL register into the AH register. This instruction extends a signed byte value into the equivalent signed word value.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| no operands | 2 | - | 1 | CBW |

Note: This instruction will set AH to 0FFh if the sign bit (bit 7) of AL is set; if bit 7 of AL is not set, AH will be set to 0. The instruction is useful for generating a word from a byte prior to performing byte division.

# CLC	Clear Carry Flag

### See also: STC, CMC, STD, CLD, STI, CLI, Flags

| Flags Affected: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | 0 |

## CLC

Logic:			CF  0

CLC clears (sets to 0) the Carry Flag. No other flags are affected.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| no operands | 2 | - | 1 | CLC |

# CLD	Clear Direction Flag

### See also: STD, STC, CLC, CMC, STI, CLI, Flags

| Flags Affected: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 |  |  |  |  |  |  |  |

## CLD

Logic:			DF  0 (Increment in string instructions)

CLD zeros the Direction Flag. No other flags are affected. Clearing the direction flag causes string operations to increment SI and DI.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| no operands | 2 | - | 1 | CLD |

Note: String instructions increment SI and DI when the direction flag is clear.

# CLI	Clear Interrupt-Enable Flag

### See also: STI, STC, CLC, CMC, STD, CLD, Flags

| Flags Affected: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 |  |  |  |  |  |  |

## CLI

Logic:			IF  0

CLI clears the Interrupt Enable Flag, suppressing processor recognition of maskable interrupts. No other flags are affected. (Non-maskable interrupts are recognized no matter what the state of the interrupt enable flag.)

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| no operands | 2 | - | 1 | CLI |

# CMC	Complement Carry Flag

### See also: STC, CLC, STD, CLD, STI, CLI, Flags

| Flags Affected: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | * |

## CMC

Logic:			CF CF

CMC reverses the current state of the Carry Flag.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| no operands | 2 | - | 1 | CMC |

# CMP	Compare

### See also: CMPS, SCAS, EA, Flags

| Flags Affected: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | * |  |  |  | * | * | * | * | * |

## CMP destination,source

Logic:		Flags set according to result of  (destination - source)

CMP compares two numbers by subtracting the source from the destination and updates the flags. CMP does not change the source or destination. The operands may be bytes or words.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
|  | byte(word) |  |  |  |
| register, register | 3 | - | 2 | CMP CX,BX |
| register, immediate | 4 | - | 3-4 | CMP BL,02h |
| accumulator, immediate | 4 | - | 2-3 | CMP AL,00010110b |
| register, memory | 9(13)+EA | 1 | 2-4 | CMP DH,ALPHA_BETA |
| memory, register | 9(13)+EA | 1 | 2-4 | CMP TOTAL,SI |
| memory, immediate | 10(14)+EA | 1 | 3-6 | CMP VALUES,3420h |

# CMPS	Compare String (Byte or Word)

### See also: CMP, CMPSB, CMPSW, SCAS, REP, CLD, STD, Flags

| Flags Affected: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | * |  |  |  | * | * | * | * | * |

## CMPS destination-string,source-string

Logic: 		CMP (DS:SI), (ES:DI)		; Sets flags only

if DF = 0

SI  SI + n		; n = 1 for byte, 2 for word

DI  DI + n

else

SI  SI - n

DI  DI - n

This instruction compares two values by subtracting the byte or word pointed to by ES:DI, from the byte or word pointed to by DS:SI, and sets the flags according to the results of the comparison. The operands themselves are not altered. After the comparison, SI and DI are incremented (if the direction flag is cleared) or decremented (if the direction flag is set), in preparation for comparing the next element of the string.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
|  | byte(word) |  |  |  |
| dest,source | 22(30) | 2 | 1 | CMPS STR1,STR2 |
| (repeat) dest,source | 9+22(30)/rep | 2/rep | 1 | REPE CMPS STR1,STR2 |

Note: This instruction is always translated by the assembler into either CMPSB, Compare String Byte, or CMPSW, Compare String Word, depending upon whether source refers to a string of bytes or words. In either case, you must explicitly load the SI and DI registers with the offset of the source and destination strings.

Example: Assuming the definition:

buffer1 db 100 dup (?)

buffer2 db 100 dup (?)

the following example compares BUFFER1 against BUFFER2 for the first mismatch.

cld				;Scan in the forward direction

mov cx, 100			;Scanning 100 bytes (CX is used by REPE)

lea si, buffer1 			;Starting address of first buffer

lea di, buffer2			;Starting address of second buffer

repe	cmps buffer1,buffer2		;	and compare it.

jne mismatch			;The Zero Flag will be cleared if there

;is a mismatch

match:			.		;If we get here, buffers match

.

mismatch:

dec si				;If we get here, we found a mismatch.

dec di				;Back up SI and DI so they point to

.		;the first mismatch

.

Upon exit from the REPE CMPS loop, the Zero Flag will be cleared if a mismatch was found, and set otherwise. If a mismatch was found, DI and SI will be pointing one byte past the byte that didn't match; the DEC DI and DEC SI backup these registers so they point to the mismatched characters.

# CMPSB	Compare String Byte

### See also: CMP, CMPS, CMPSW SCAS, REP, CLD, STD, Flags

| Flags Affected: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | * |  |  |  | * | * | * | * | * |

## CMPSB

Logic:	 	CMP (DS:SI), (ES:DI) 		; Sets flags only

if DF = 0

SI  SI + 1

DI  DI + 1

else

SI  SI - 1

DI  DI - 1

This instruction compares two values by subtracting the byte pointed to by ES:DI, from the byte pointed to by DS:SI, and sets the flags according to the results of the comparison. The operands themselves are not altered. After the comparison, SI and DI are incremented (if the direction flag is cleared) or decremented (if the direction flag is set), in preparation for comparing the next element of the string.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| - | 22 | 2 | 1 | CMPSB |
| (repeat) | 9+22/rep | 2/rep | 1 | REPE CMPSB |

Example: The following example compares BUFFER1 against BUFFER2 for the first mismatch.

cld				;Scan in the forward direction

mov cx, 100			;Scanning 100 bytes (CX is used by REPE)

lea si, buffer1			;Starting address of first buffer

lea di, buffer2			Starting address of second buffer

repe	cmpsb				;...and compare it.

jne mismatch			;The Zero Flag will be cleared if there

;is a mismatch

match:			.		;If we get here, buffers match

.

mismatch:

dec si				;If we get here, we found a mismatch.

dec di				;Back up SI and DI so they point to the

.		;first mismatch

Upon exit from the REPE CMPSB loop, the Zero Flag will be cleared if a mismatch was found, and set otherwise. If a mismatch was found, DI and SI will be pointing one byte past the byte that didn't match; the DEC DI and DEC SI instructions backup these registers so they point to the mismatched characters.

# CMPSW	Compare String Word

### See Also: CMP, CMPS, CMPSB, SCAS, REP, CLD, STD, Flags

| Flags Affected: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | * |  |  |  | * | * | * | * | * |

## CMPSW

Logic: 	CMP (DS:SI), (ES:DI)	; Sets flags only

if DF = 0

SI  SI + 2

DI  DI + 2

else

SI  SI - 2

DI  DI - 2

This instruction compares two numbers by subtracting the word pointedto by ES:DI, from the word pointed to by DS:SI, and sets the flags according to the results of the comparison. The operands themselves are not altered. After the comparison, SI and DI are incremented (if the direction flag is cleared) or decremented (if the direction flagis set), in preparation for comparing the next element of the string.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| - | 30 | 21 | CMPSW |  |
| (repeat) | 9 + 30/rep | 2/rep | 1 | REPE CMPSW |

Example: The following example compares BUFFER1 against BUFFER2 for the first mismatch.

cld			;Scan in the forward direction

mov cx, 50 		;Scanning 50 words (100 bytes)

lea si, buffer1		;Starting address of first buffer

lea di, buffer2		;Starting address of second buffer

repe	cmps			;...and compare it.

jne mismatch		;The Zero Flag will be cleared if there

;is a mismatch

match:		.		;If we get here, buffers match

.

mismatch:

dec si 			;If we get here, we found a mismatch.

dec si 			;Back up DI and SI so they point to the

dec di	 		;…first mismatch

dec di

Upon exit from the REPE CMPSW loop, the Zero Flag will be cleared if a mismatch was found, and set otherwise. If a mismatch was found, DI and SI will be pointing one word (two bytes) past the word that didn't match; the DEC DI and DEC SI pairs backup these registers so they point to the mismatched characters.

# CWD	Convert Word to Doubleword

### See Also: CBW, DIV, IDIV

Flags: not altered

## CWD

Logic:	 	if (AX < 8000h) then

DX  0

else

DX  FFFFh

CWD extends the sign bit of the AX register into the DX register. This instruction generates the double-word equivalent of the signed number in the AX register.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| no operands | 5 | - | 1 | CWD |

Note: 	This instruction will set DX to 0FFFFh if the sign bit (bit 15) of AX is set; if bit 15 of AX is not set, DX will be set to 0.

# DAA	Decimal Adjust after Addition

### See Also: DAS, AAA, AAS, AAM, AAD, Flags

| Flags Affected: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ? |  |  |  | * | * | * | * | * |

## DAA

Logic:	 	If (AL & 0Fh) > 9 or (AF = 1) then

AL  AL + 6

AF  1

else

AF  0

If (AL > 9Fh) or (CF = 1) then

AL  AL + 60h

CF  1

else

CF  0

DAA corrects the result of a previous addition of two valid packed decimal operands (note that this result must be in AL). This instruction changes the content of AL so that it will contain a pair of valid packed decimal digits.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| no operands | 4 | - | 1 | DAA |

Note: Packed BCD stores one digit per nibble (4 bits); the least significant digit is in the lower nibble.  It is not possible to apply an adjustment after division or multiplication of packed BCD numbers. If you need to use multiplication or division, it is better to use unpacked BCD numbers.See, for example, the description of AAM (ASCII Adjust after Multiply).

# DAS	Decimal Adjust after Subtraction

### See Also: AAS, DAA, SUB, SBB, DEC, NEG, Flags

| Flags Affected: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ? |  |  |  | * | * | * | * | * |

## DAS

Logic: 	If(AL & 0Fh) > 9 or (AF = 1) then

AL  AL - 6

AF  1

else

AF  0

If (AL > 9Fh) or (CF = 1) then

AL  AL - 60h

CF  1

else

CF  0

DAS corrects the result of a previous subtraction of two valid packed decimal operands (note that this result must be in AL). This instruction changes the content of AL so that it will contain a pair of valid packed decimal digits.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| no operands | 4 | - | 1 | DAS |

Note:	Packed BCD stores one digit per nibble (4 bits); the least significant digit is in the lower nibble.

It is not possible to apply an adjustment after division or multiplication of packed BCD numbers. If you need to use multiplication and division, it is better to use unpacked BCD numbers. See, for example, the description of AAM (ASCII Adjust after Multiply).

# DEC	Decrement

### See Also: SUB, SBB, AAS, DAS, EA, Flags

| Flags Affected: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | * |  |  |  | * | * | * | * |  |

## DEC destination

Logic:		destination  destination - 1

This instruction decrements the destination by one. The destination operand, which may be either a word or a byte, is treated as an unsigned binary number.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| reg16 | 2 | - | 1 | DEC BX |
| reg8 | 3 | - | 2 | DEC BL |
| memory | 15(23)+EA | 2 | 2-4 | DEC MATRIX[SI] |

Note: This instruction does not set the carry, so if you need to decrement a multi-word number, it is better to use the SUB and SBB instructions.

# DIV	Divide, Unsigned

### See Also: IDIV, SHR, AAD, CBW, CWD, INT 00h, EA, Flags

| Flags Affected: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ? |  |  |  | ? | ? | ? | ? | ? |

## DIV source

Logic:		AL  AX / source		;Source is byte

AH  remainder

or

AX  DX:AX / source		;Source is word

DX  remainder

This instruction performs unsigned division. If the source is a byte, DIV divides the word value in AX by source, returning the quotient in AL and the remainder in AH. If the source is a word, DIV divides the double-word value in DX:AX by the source, returning the quotient in AX and the remainder in DX.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| reg8 | 0-90 | - | 2 | DIV BL |
| reg16 | 4-162 | - | 2 | DIV BX |
| mem8 | (86-96)+EA | 1 | 2-4 | DIV VYUP |
| mem16 | (154-172)+EA | 1 | 2-4 | DIV NCONQUER[SI] |

Note: If the result is too large to fit in the destination (AL or AX), an INT 0 (Divide by Zero) is generated, and the quotient and remainder are undefined.

When an Interrupt 0 (Divide by Zero) is generated, the saved CS:IP value on the 80286 and 80386 points to the instruction that failed (the DIV instruction).On the 8088/8086, however, CS:IP points to the instruction following the failed DIV instruction.

# ESC	Escape

### See Also: HLT, WAIT, LOCK, EA

Flags: not altered

## ESC coprocessor's-opcode, source

ESC is used to pass control from the microprocessor to a coprocessor, such as an 8087 or 80287. In response to ESC, the microprocessor accesses a memory operand -the instruction for the coprocessor-and places it on the bus. The coprocessor watches for ESC commands and executes the instruction placed on the bus, using the effective address of source.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
|  | byte(word) |  |  |  |
| immediate, memory | 8(12)+ EA | 1 | 2-4 | ESC 6,ADR[SI] |
| immediate, register | 2 | - | 2 | ESC COPROC-CODE,AH |

Note: In order to synchronize with the math coprocessor, the programmer must explicitly code the WAIT instruction preceding all ESC instructions. The 80286 and 80386 have automatic instruction synchronization, hence WAITs are not needed.

# HLT	Halt the Processor

### See also: WAIT, ESC, LOCK

Flags: not altered

## HLT

This instruction halts the CPU. The processor leaves the halted state in response to a non-maskable interrupt; a maskable interrupt with interrupts enabled; or activation of the reset line.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| no operands | 2 | - | 1 | HLT |

# IDIV	Integer Divide, Signed

### See also: DIV, SAR, AAD, CBW, CWD, INT 00h, EA, Flags

| Flags Affected: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ? |  |  |  | ? | ? | ? | ? | ? |

## IDIV source

Logic:			AL  AX / source		; Byte source

AH  remainder

or

AX  DX:AX / source		; Word source

DX  remainder

IDIV performs signed division. If source is a byte, IDIV divides the word value in AX by source, returning the quotient in AL and the remainder in AH. If source is a word, IDIV divides the double-word value in DX:AX by source, returning the quotient in AX and the remainder in DX.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| reg8 | 101-112 | - | 2 | IDIV CL |
| reg16 | 165-184 | - | 2 | IDIV DX |
| mem8 | (107-118)+EA | 1 | 2-4 | IDIV BYTE[SI] |
| mem16 | (175-194)+EA | 1 | 2-4 | IDIV [BX].WORD_ARRAY |

Note: If the result is too large to fit in the destination (AL or AX), an INT 0 (Divide by Zero) is generated, and the quotient and remainder are undefined.

The 80286 and 80386 microprocessors are able to generate the largest negative number (80h or 8000h) as a quotient for this instruction, but the 8088/8086 will generate an Interrupt 0 (Divide by Zero) if this situation occurs.

When an Interrupt 0 (Divide by Zero) is generated, the saved CS:IP value on the 80286 and 80386 points to the instruction that failed (the IDIV instruction). On the 8088/8086, however, CS:IP points to the instruction following the failed IDIV instruction.

# IMUL	Integer Multiply, Signed

### See Also: MUL, AAM, EA, Flags

| Flags Affected: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | * |  |  |  | ? | ? | ? | ? | * |

## IMUL source

Logic: 		AX AL * source		; if source is a byte

or

DX:AX  AX * source		; if source is a word

IMUL performs signed multiplication. If source is a byte, IMUL multiplies source by AL, returning the product in AX. If source is a word, IMUL multiplies source by AX, returning the product in DX:AX. The Carry Flag and Overflow Flag are set if the upper half of the result (AH for a byte source, DX for a word source) contains any significant digits of the product, otherwise they are cleared.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| reg8 | 80-98 | - | 2 | IMUL CL |
| reg16 | 128-154 | - | 2 | IMUL BX |
| mem8 | (86-104)+EA | 1 | 2-4 | IMUL BITE |
| mem16 | (138-164)+EA | 1 | 2-4 | IMUL WORD[BP][DI] |

# IN	Input Byte or Word

### See Also: OUT

Flags: not altered

## IN accumulator,port

Logic:		accumulator  (port)

IN transfers a byte or a word from a port to AL or AX. The port may be specified by an immediate byte value (for ports 0 through 255) or by the DX register (allowing access to all ports).

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
|  | byte(word) |  |  |  |
| accumulator, immed8 | 10(14) | 1 | 2 | IN AL,45h |
| accumulator, DX | 8(12) | 1 | 1 | IN AX,DX |

Note: It is advised that hardware not use I/O ports F8h through FFh, since these are reserved for controlling the math coprocessor and future processor extensions.

# INC	Increment

### See Also: ADD, ADC, AAA, DAA, DEC, EA, Flags

| Flags Affected: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | * |  |  |  | * | * | * | * |  |

## INC destination

Logic:		destination  destination + 1

INC adds 1 to the destination. The destination, which may be either a byte or a word, is considered an unsigned binary number.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| byte(word) |  |  |  |  |
| reg16 | 2 | - | 1 | INC BX |
| reg8 | 3 | - | 2 | INC BL |
| memory | 15(23)+EA | 2 | 2-4 | INC THETA[BX] |

Note: This instruction does not set the carry flag.If you need to add 1 to a multi-word number, it is better to use the ADD and ADC instructions instead.

# INT	Interrupt

### See also:  IRET  INTO  PUSHF  CALL  PUSHF  INT 03h  Flags

| Flags Affected: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 0 |  |  |  |  |  |  |  |

## INT interrupt-num

Logic:			PUSHF				;Push flags onto stack

TF  0				;Clear Trap Flag

IF  0				;Disable Interrupts

CALL FAR (INT*4)		;Call the interrupt handler

INT pushes the flags register, clears the Trap and Interrupt-enable Flags, pushes CS and IP, then transfers control to the interrupt handler specified by the interrupt-num.If the interrupt handler returns using an IRET instruction, the original flags are restored.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
|  | byte(word) |  |  |  |
| immed8 (type=3) | 52 | 5 | 1 | INT 3 |
| immed8 (type<>3) | 51 | 5 | 2 | INT 21 |

Note: The flags are stored in the same format as that used  by the PUSHF instruction. The address of the interrupt vector is determined by multiplying the interrupt-num by 4. The first word at the resulting address is loaded into IP, and the second word into CS.

All interrupt-nums except type 3 generate a two-byte  opcode; type 3 generates a one-byte instruction called the Breakpoint interrupt.

# INTO	Interrupt on Overflow

### See Also: INT, IRET, JNO, JO, PUSHF, CALL, INT 04h, Flags

| Flags Affected: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 0 |  |  |  |  |  |  |  |

## INTO

Logic:		if (OF = 1)

PUSHF			;Push flags onto stack

TF  0			;Clear Trap Flag

IF  0				;Disable Interrupts

CALL FAR (10h)	;The INTO vector is at 0000:0010h

INTO activates interrupt type 4 if the Overflow Flag is set; otherwise it does nothing. This interrupt operates identically to an "INT 4" if the overflow flag is set, in which case INTO pushes the flags register, clears the Trap and Interrupt-enable Flags, pushes CS and IP, then transfers control to the interrupt-num 4 handler, which is pointed to by the vector at location 10h.If the interrupt handler returns using an IRET instruction, the original flags are restored.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
|  | byte(word) |  |  |  |
| no operands | 53 or 4 | 5 | 1 | INTO |

Note: The flags are stored in the same format as that used by the PUSHF instruction.  INTO can be used after an operation that may cause overflow, to call a recovery procedure.

# IRET	Interrupt Return

### See Also: INT, INTO, POP, POPF

| Flags Affected: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | r | r | r | r | r | r | r | r | r |

## IRET

Logic:			POP IP

POP CS

POPF		; Pop flags from stack

IRET returns control from an interrupt routine to the point where the interrupt occurred, by popping IP, CS, and the Flag registers.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| no operands | 32 | 3 | 1 | IRET |

# JA	Jump If Above

### See Also: JNBE, JAE, JG, JBE

Flags: not altered

## JA short-label

Jump Condition:		Jump if CF = 0 and ZF = 0

Used after a CMP or SUB instruction, JA transfers control to short- label if the first operand (which should be unsigned) was greater than the second operand (also unsigned). The target of the jump must be within -128 to +127 bytes of the next instruction.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| short-label | 16 or 4 | - | 2 | JA ABOVE |

Note: JNBE, Jump Not Below or Equal, is the same instruction as JA.

JA, Jump on Above, should be used when comparing unsigned numbers.

JG, Jump on Greater, should be used when comparing signed numbers.

# JAE	Jump If Above or Equal

### See Also: JNB, JA, JGE, JB

Flags: not altered

## JAE short-label

Jump Condition:		Jump if CF = 0

Used after a CMP or SUB instruction, JAE transfers control to short- label if the first operand (which should be unsigned) was greater than or equal to the second operand (also unsigned). The target of the jump must be within -128 to +127 bytes of the next instruction.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| short-label | 16 or 4 | - | 2 | JAE ABOVE_EQUAL |

Note: JNB (Jump Not Below) is the same instruction as JAE.

JAE, Jump on Above or Equal, should be used whencomparing unsigned numbers.

JGE, Jump on Greater or Equal, should be used when comparing signed numbers.

# JB	Jump If Below

### See Also: JNAE, JC, JL, JAE

Flags: not altered

## JB short-label

Jump Condition:		Jump if CF = 1

Used after a CMP or SUB instruction, JB transfers control to short- label if the first operand was less than the second.(Both operands are treated as unsigned numbers.)The target of the jump must be within -128 to +127 bytes of the next instruction.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| short-label | 16 or 4 | - | 2 | JB BELOW |

Note: JC (Jump if Carry), JB, and JNAE (Jump if Not Above or Equal), are all synonyms for the same instruction.

JB, Jump if Below, should be used when comparing unsigned numbers.

JL, Jump if Less Than, should be used when comparing signed numbers.

# JBE	Jump If Below or Equal

### See Also: JNA, JLE, JA

Flags: not altered

## JBE short-label

Jump Condition:		Jump if CF = 1 or ZF = 1

Used after a CMP or SUB instruction, JBE transfers control to short- label if the first operand was less than or equal to the second. (Both operands are treated as unsigned numbers.)The target of the jump must be within -128 to +127 bytes of the next instruction.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| short-label | 16 or 4 | - | 2 | JBE NOT_ABOVE |

Note: JNA, Jump if Not Above, is the same instruction as JBE.

JBE, Jump if Below or Equal, should be used when comparing unsigned numbers.

JLE, Jump if Less Than or Equal, should be used when comparing signed numbers.

# JC	Jump If Carry

### See Also: JB, JNAE, JNC

Flags: not altered

## JC short-label

Jump Condition:		Jump if CF = 1

JC transfers control to short-label if the Carry Flag is set. The target of the jump must be within -128 to +127 bytes of the next instruction.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| short-label | 16 or 4 | - | 2 | JC CARRY_SET |

Note: JB (Jump if Below), JC, and JNAE (Jump if Not Above or Equal) are all synonyms for the same instruction.

Use JNC, Jump if No Carry, to jump if the carry flag is clear.

# JCXZ	Jump If CX Register Zero

### See Also: LOOP, LOOPE, LOOPZ, LOOPNZ, LOOPNE

Flags: not altered

## JCXZ short-label

Jump Condition:		Jump if CX = 0

JCXZ transfers control to short-label if the CX register is 0. The target of the jump must be within -128 to +127 bytes of the nextinstruction.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| short-label | 18 or 6 | - | 2 | JCXZ COUNT_DONE |

Note: This instruction is commonly used at the beginning of a loop to bypass the loop if the counter variable (CX) is at 0.

# JE	Jump If Equal

### See Also: JZ, JNE

Flags: not altered

## JE short-label

Jump Condition: 		Jump if ZF = 1

Used after a CMP or SUB instruction, JE transfers control to short- label if the first operand is equal to the second operand. The target of the jump must be within -128 to +127 bytes of the next instruction.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| short-label | 16 or 4 | - | 2 | JE ZERO |

Note: JZ, Jump if Zero, is the same instruction as JE

# JG	Jump If Greater

### See also: JNLE, JA, JLE

Flags: not altered

## JG short-label

Jump Condition:		 Jump if ZF = 0 and SF = OF

Used after a CMP or SUB instruction, JG transfers control to short- label if the first operand is greater than the second. (Both operands are treated as signed numbers.)The target of the jump must be within -128 to +127 bytes of the next instruction.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| short-label | 16 or 4 | - | 2 | JG GREATER |

Note: JNLE, Jump if Not Less or Equal, is the same instruction as JG.

JA, Jump if Above, should be used when comparing unsigned numbers.

JG, Jump if Greater, should be used when comparing signed numbers.

# JGE	Jump If Greater or EQual

### See also: JNL, JAE, JL

Flags: not altered

## JGE short-label

Jump Condition:		Jump if SF = OF

Used after a CMP or SUB instruction, JGE transfers control to short- label if the first operand is greater than or equal to the second. (Both operands are treated as signed numbers.)The target of the jump must be within -128 to +127 bytes of the next instruction.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| short-label | 16 or 4 | - | 2 | JGE GREATER_EQUAL |

Note: JNL, Jump if Not Less, is the same instruction as JGE.

JAE, Jump if Above or Equal, should be used when comparing unsigned numbers.

JGE, Jump if Greater or Equal, should be used when comparing signed numbers.

# JL	Jump If Less

### See Also: JNGE, JB, JGE

Flags: not altered

## JL short-label

Jump Condition:		Jump if SF <> OF

Used after a CMP or SUB instruction, JL transfers control to short- label if the first operand is less than the second. (Both operands are treated as signed numbers.)The target of the jump must be within - 128 to +127 bytes of the next instruction.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| short-label | 16 or 4 | - | 2 | JL LESS |

Note: JNGE, Jump if Not Greater or Equal, is the same instruction as JL.

JB, Jump if Below, should be used when comparing unsigned numbers.

JL, Jump if Less, should be used when comparing signed numbers.

# JLE	Jump If Less or Equal

### See Also: JNG, JBE, JNA, JG

Flags: not altered

## JLE short-label

Jump Condition:		Jump if SF <> OF or ZF = 1

Used after a CMP or SUB instruction, JLE transfers control to short- label if the first operand is less than or equal to the second . (Both operands are treated as signed numbers.)The target of the jump must be within -128 to +127 bytes of the next instruction.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| short-label | 16 or 4 | - | 2 | JLE NOT_GREATER |

Note: JNG, Jump if Not Greater, is the same instruction as JLE.

JBE, Jump if Below or Equal, should be used when comparing unsigned numbers.

JLE, Jump if Less or Equal, should be used when comparing signed numbers.

# JMP	Jump Unconditionally

### See also:CALL, RET, SHORT, NEAR, FAR, PROC, EA

Flags: not altered

## JMP target

Jump Condition:		Jump always

JMP always transfer control to the target location. Unlike CALL, JMP does not save IP, because no RETurn is expected. An intrasegment JMP may be made either through memory or through a 16-bit register; an intersegment JMP can be made only through memory.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| short-label | 15 | - | 2 | JMP ROPE_NEAR |
| near-label | 15 | - | 3 | JMP SAME_SEGMENT |
| far-label | 15 | - | 5 | JMP FAR_LABEL |
| memptr16 | 18+EA | - | 2-4 | JMP SAME_SEG |
| regptr16 | 11 | - | 2 | JMP BX |
| memptr32 | 24+EA | - | 2-4 | JMP NEXT_SEG |

Note: If the assembler can determine that the target of an intrasegment jump is within 127 bytes of the current location, the assembler will automatically generate a short-jump (two-byte) instruction; otherwise, a 3- byte NEAR JMP is generated.

You can force the generation of a short jump by explicit use of the operator "short," as in:

JMPshort near_by

# JNA	Jump If Not Above

### See Also: JBE, JLE

Flags: not altered

## JNA short-label

JNA is a synonym for JBE. See the description for JBE.

# JNAE	Jump If Not Above or Equal

### See Also: JB, JL

Flags: not altered

## JNAE short-label

JNAE is a synonym for JB. See the description for JB.

# JNB	Jump If Not Below

### See Also: JAE, JGE

Flags: not altered

## JNB short-label

JNB is a synonym for JAE. See the description for JAE.

# JNBE	Jump If Not Below or Equal

### See Also: JA, JG

Flags: not altered

## JNBE short-label

JNBE is a synonym for JA. See the description for JA.

# JNC	Jump If No Carry

### See Also: JC

Flags: not altered

## JNC short-label

Jump Condition:		Jump if CF = 0

JNC transfers control to short-label if the Carry Flag is clear. The target of the jump must be within -128 to +127 bytes of the next instruction.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| short-label | 16 or 4 | - | 2 | JNC CARRY_CLEAR |

Note: Use JC, Jump if Carry, to jump if the carry flag is set.

# JNE	Jump If Not Equal

### See Also: JNZ, JE

Flags: not altered

## JNE short-label

Jump Condition:		Jump if ZF = 0

Used after a CMP or SUB instruction, JNE transfers control to short- label if the first operand is not equal to the second. The target of the jump must be within -128 to +127 bytes of the next instruction.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| short-label | 16 or 4 | - | 2 | JNE NOT_EQUAL |

Note: JNZ, Jump if Not Zero, is the same instruction as JNE.

# JNG	Jump If Not Greater

### See Also: JLE, JBE

Flags: not altered

## JNG short-label

JNG is a synonym for JLE. See the description for JLE.

# JNGE	Jump If Not Greater or Equal

### See Also: JLJB

Flags: not altered

## JNGE short-label

JNGE is a synonym for JL. See the description for JL.

# JNL	Jump If Not Less

### See Also: JGE, JAE

Flags: not altered

## JNL short-label

JNL is a synonym for JGE. See the description for JGE.

# JNLE	Jump If Not Less or Equal

### See Also: JG, JA

Flags: not altered

## JNLE short-label

JNLE is a synonym for JG. See the description for JG.

# JNO	Jump If No Overflow

### See Also: JO, INTO

Flags: not altered

## JNO short-label

Jump Condition:		Jump if OF = 0

JNO transfers control to short-label if the Overflow Flag is clear. The target of the jump must be within -128 to +127 bytes of the next instruction.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| short-label | 16 or 4 | - | 2 | JNO NO_OVERFLOW |

Note: Use JO, Jump if Overflow, to jump if the overflow flag is set.

# JNP	Jump If No Parity

### See Also: JP, OJP

Flags: not altered

## JNP short-label

Jump Condition:		Jump if PF = 0

JNP transfers control to short-label if the Parity Flag is clear. The target of the jump must be within -128 or +127 bytes of the next instruction.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| short-label | 16 or 4 | - | 2 | JNP ODD_PARITY |

Note: JPO, Jump if Parity Odd, is the same instruction a JNP.

Use JP, Jump on Parity, to jump if the parity flag is set.

# JNS	Jump If No Sign

### See Also: JS

Flags: not altered

## JNS short-label

Jump Condition:Jump if SF = 0

JNS transfers control to short-label if the Sign Flag is clear. The target of the jump must be within -128 to +127 bytes of the next instruction.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| short-label | 16 or 4 | - | 2 | JNS AQUARIUS |

Note: Use JS, Jump if Sign, to jump if the sign flag is set.

# JNZ	Jump If Not Zero

### See Also: JNE, JE

Flags: not altered

## JNZ short-label

JNZ is a synonym for JNE. See the description for JNE.

# JO	Jump If Overflow

### See Also: JNO, INTO

Flags: not altered

## JO short-label

Jump Condition:		Jump if OF = 1

JO transfers control to short-label if the Overflow Flag is set.The target of the jump must be within -128 to +127 bytes of the next instruction.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| short-label | 16 or 4 | - | 2 | JO SIGNED_OVERFLOW |

Note: Use JNO, Jump if No Overflow, to jump if the overflow flag is clear.

# JP	Jump If Parity

### See Also: JPE, JNP

Flags: not altered

## JP short-label

Jump Condition:		Jump if PF = 1

JP transfers control to short-label if the Parity Flag is set. The target of the jump must be within -128 to +127 bytes of the next instruction.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| short-label | 16 or 4 | - | 2 | JP EVEN_PARITY |

Note: JPE, Jump if Parity Even, is the same instruction as JP.

Use JNP, Jump if No Parity, to jump if the Parity Flag is clear.

# JPE	Jump If Parity Even

### See Also: JP, JNP

Flags: not altered

## JPE short-label

JPE is a synonym for JP. See the description for JP.

# JPO	Jump If Parity Odd

### See also: JNP, JP

Flags: not altered

## JPO short-label

JPO is a synonym for JNP. See the description for JNP.

# JS	Jump If Sign

### See also: JNS

Flags: not altered

## JS short-label

Jump Condition:		Jump if SF = 1

JS transfers control to short-label if the Sign Flag is set. The target of the jump must be within -128 to +127 bytes of the next instruction.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| short-label | 16 or 4 | - | 2 | JS NEGATIVE |

Note: Use JNS, Jump if No Sign, to jump if the sign flag is clear.

# JZ	Jump If Zero

### See also: JE, JNE

Flags: not altered

## JZ short-label

JZ is a synonym for JE. See the description for JE.

# LAHF	Load Register AH from Flags

### See also: SAHF, PUSHF, POPF

Flags: not altered

## LAHF

Logic:		AH bitsFlag-reg bits

7 6 4 2 0 S Z A P C

LAHF copies the five 8080/8085 flags (Sign, Zero, Auxiliary Carry, Parity, and Carry) into bits 7, 6, 4, 2, and 0, respectively, of the AH register. The flags themselves are unchanged by this instruction.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| no operands | 4 | - | 1 | LAHF |

Note:	This instruction is primarily used to provide upward compatibility between the 8080/8085 family and the 8086 family.

After this instruction is executed, bits 1, 3 and 5 of AH are undefined.

# LDS	Load Pointer using DS

### See also: LEA, LES, OFFSET, EA

Flags: not altered

## LDS	destination,source

Logic:		DS  (source + 2)

destination  (source)

LDS loads into two registers the 32-bit pointer variable found in memory at source. LDS stores the segment value (the higher order word of source) in DS and the offset value (the lower order word of source) in the destination register. The destination register may be any any 16-bit general register (that is, all registers except segment registers).

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| reg16, mem32 | 24+EA | 2 | 2-4 | LDS DI,32_POINTER |

Note: LES, Load Pointer Using ES, is a comparable instruction that loads the ES register rather than the DS register.

# LEA	Load Effective Address

### See Also: LDS, LES, OFFSET, EA

Flags: not altered

## LEA destination,source

Logic:		destination  Addr(source)

LEA transfers the offset of the source operand (rather than its value) to the destination operand. The source must be a memory reference, and the destination must be a 16-bit general register.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| reg16, mem16 | 2+EA | - | 2-4 | LEA BX,MEM_ADDR |

Note: This instruction has an advantage over using the OFFSET operator with the MOV instruction, in that the source operand can be subscripted. For example, this is legal:

LEA BX, TABLE[SI]			;Legal

whereas

MOV BX, OFFSET TABLE[SI]		;Not legal

is illegal, since the OFFSET operator performs its calculation at assembly time and this address is not known until run time.

Example: The DOS print string routine, Function 09h, requires a pointer to the string to be printed in DS:DX. If the string you wished to print was at address "PRINT-ME" in the same data segment, you could load DS:DX with the required pointer using this instruction:

LEA DX, PRINT-ME

# LES	Load Pointer using ES

### See Also: LEA, LDS, OFFSET, EA

Flags: not altered

## LES dest-reg,source

Logic:		ES  (source)

dest-reg  (source + 2)

LES loads into two registers the 32-bit pointer variable found in memory at source. LES stores the segment value (the higher order word of source) in ES and the offset value (the lower order word of source) in the destination register. The destination register may be any any 16-bit general register (that is, all registers except segment registers).

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| reg16, mem32 | 24+EA | 2 | 2-4 | LES DI, STR_ADDR |

Note: LDS, Load Pointer Using DS, is a comparable instruction that loads the DS register rather than the ES register.

# LOCK	Lock the Bus

### See Also: HLT, WAIT, ESC

Flags: not altered

## LOCK

LOCK is a one-byte prefix that can precede any instruction. LOCK causes the processor to assert its bus lock signal while the instruction that follows is executed. If the system is configured such that the LOCK signal is used, it prevents any external device or event from accessing the bus, including interrupts and DMA transfers.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| no operands | 2 | - | 1 | LOCK XCHG FLAG,AL |

Note: This instruction was provided to support multiple processor systems with shared resources. In such a system, access to those resources is generally controlled via a software-hardware combination using semaphores.

This instruction should only be used to prevent other bus masters from interrupting a data movement operation. This prefix should only be used with XCHG, MOV, and MOVS.

# LODS	Load String (Byte or Word)

### See also: LODSB, LODSW, CMPS, MOVS, SCAS, STOS, REP, CLD

Flags: not altered

## LODS source-str

Logic:		Accumulator  (DS:SI)

if DF = 0

SI  SI + n ; n = 1 for byte, 2 for word scan

else

SI  SI - n

LODS transfers the value (word or byte) pointed to by DS:SI into AX or AL.It also increments or decrements SI (depending on the state of the Direction Flag) to point to the next element.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
|  | byte(word) |  |  |  |
| source-str | 12(16) | - | 1 | LODS LIST |
| (repeat) source-str | 9+13(17)/rep | 1/rep | 1 | REP LODS LIST |

Note: This instruction is always translated by the assembler into either LODSB, Load String Byte, or LODSW, Load String Word, depending upon whether source-str refers to a string of bytes or words. In either case, however, you must explicitly load the SI register with the offset of the string.

Although it is legal to repeat this instruction, it is almost never done since doing so would continually overwrite the value in AL.

Example: The following example sends the eight bytes at INIT_PORT to port 250. (Don't try this on your machine, unless you know what's hanging off port 250.)

INIT_PORT:

DB	'$CMD0000'			;The string we want to send

.

.

CLD					;Move forward through string at INIT_PORT

LEA SI,INIT_PORT			;SI gets starting address of string

MOV CX,8				;CX is counter for LOOP instruction

AGAIN:

LODS INIT_PORT			;"INIT_PORT" is needed only by the

OUT 250,AL				;assembler, for determining word or byte

LOOP AGAIN

# LODSB	Load String Byte

### See Also: LODS, LODSW, CMPS, MOVS, SCAS, STOS, REP, CLD, STD

Flags: not altered

## LODSB

Logic:			AL  (DS:SI)

if DF = 0

SI  SI + 1

else

SI  SI - 1

LODSB transfers the byte pointed to by DS:SI into AL and increments or decrements SI (depending on the state of the Direction Flag) to point to the next byte of the string.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| - | 12 | - | 1 | LODSB |
| (repeat) | 9+13/rep | 1/rep | 1 | REP LODSB |

Note: Although it is legal to repeat this instruction, it is almost never done since doing so would continually overwrite the value in AL.

Example: The following example sends the eight bytes at INIT_PORT to port 250. (Don't try this on your machine, unless you know what's hanging off port 250.)

INIT_PORT:

DB '$CMD0000'			;The string we want to send

.

.

CLD				;Move forward through string at INIT_PORT

LEA SI, INIT_PORT		;SI gets starting address of string

MOV CX, 8			;CX is counter for LOOP instruction

AGAIN:

LODSB				;Load a byte into AL...

OUT 250,AL			; ...and output it to the port.

LOOP AGAIN

# LODSW	Load String Word

### See also: LODS, LODSB, CMPS, MOVS, SCAS, STOS, REP, CLD, STD

Flags: not altered

## LODSW

Logic:		AX  (DS:SI)

if DF = 0

SI  SI + 2

else

SI  SI - 2

LODSW transfers the word pointed to by DS:SI into AX and increments or decrements SI (depending on the state of the Direction Flag) to point to the next word of the string.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| - | 16 | - | 1 | LODSW |
| (repeat) | 9+17/rep | 1/rep | 1 | REP LODSW |

Note: Although it is legal to repeat this instruction, it is almost never done since doing so would continually overwrite the value in AL.

Example: The following example sends the eight bytes at INIT_PORT to port 250. (Don't try this on your machine, unless you know what's hanging off port 250.)

INIT_PORT:

DB'$CMD0000'				;The string we want to send

.

.

CLD					;Move forward through string at INIT_PORT

LEA SI, INIT_PORT			;SI gets starting address of string

MOV CX, 4				;Moving 4 words (8 bytes)

AGAIN:

LODSW				;Load a word into AX...

OUT 250,AX				; ...and output it to the port.

LOOP AGAIN

# LOOP	Loop on Count

### See Also: LOOPE, LOOPNE, LOOPNZ, LOOPZ, JCXZ

Flags: not altered

## LOOP short-label

Logic:			CX  CX - 1

If (CX <> 0)

JMP short-label

LOOP decrements CX by 1, then transfers control to short-label if CX is not 0.Short-label must be within -128 to +127 bytes of the next instruction.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| short-label | 17/5 | - | 2 | LOOP AGAIN |

# LOOPE	Loop While Equal

### See also: LOOP, LOOPNE, LOOPNZ, LOOPZ, JCXZ

Flags: not altered

## LOOPE short-label

Logic:			CX  CX - 1

If CX <> 0 and ZF = 1

JMP short-label

Used after a CMP or SUB, LOOPE decrements CX by 1, then transfers control to short-label if the first operand of the CMP or SUB is equal to the second operand. Short-label must be within -128 to +127 bytes of the next instruction.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| short-label | 18 or 6 | - | 2 | LOOPE AGAIN |

Note: LOOPZ, Loop if Zero, is the same instruction.

# LOOPNE	Loop While not Equal

### See also: LOOPZ, LOOP, LOOPE, LOOPNZ, JCXZ

Flags: not altered

## LOOPNE short-label

Logic:			CX  CX - 1

If CX <> 0 and ZF = 0

JMP short-label

Used after a CMP or SUB, LOOPNE decrements CX by 1, then transfers control to short-label if the first operand of the CMP or SUB is not equal to the second operand. Short-label must be within -128 to +127 bytes of the next instruction.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| short-label | 19 or 5 | - | 2 | LOOPNE AGAIN |

Note: LOOPNZ, Loop While Not Zero, is the same instruction.

# LOOPNZ	Loop While not Zero

### See Also: LOOPNE, LOOP, LOOPE, JCXZ

Flags: not altered

## LOOPNZ short-label

LOOPNZ is a synonym for LOOPNE. See the description for LOOPNE.

# LOOPZ	Loop While Zero

### See also: LOOPE, LOOP, LOOPNE, JCXZ

Flags: not altered

## LOOPZ short-label

LOOPZ is a synonym for LOOPE. See the description for LOOPE.

# MOV	Move (Byte or Word)

### See also: MOVSPUSHPOPXCHGXLATEA

Flags: not altered

## MOV destination,source

Logic:			destination  source

MOV copies a byte or word from the source into the destination.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
|  | byte(word) |  |  |  |
| register, register | 2 | - | 2 | MOV BX,CX |
| memory, accumulator | 10(14) | 1 | 3 | MOV MEM_DEST,AL |
| accumulator, memory | 10(14) | 1 | 3 | MOV AX,MEM_SOURCE |
| memory, register | 9(13)+EA | 1 | 2-4 | MOV MEM_DEST,BX |
| register, memory | 8(12)+EA | 1 | 2-4 | MOV BX,MEM_SOURCE |
| register, immediate | 4 | - | 2-3 | MOV BX,0F6CDh |
| memory, immediate | 10(14)+EA | 1 | 3-6 | MOV MASK,0F6CDh |
| seg-reg, reg16 | 2 | - | 2 | MOV DS,BX |
| seg-reg, mem16 | 8(12)+EA | 1 | 2-4 | MOV DS,SEGMENT_VAL |
| reg16, seg-reg | 2 | - | 2 | MOV BP,SS |
| memory, seg-reg | 9(13)+EA | 1 | 2-4 | MOV CODE_VAR,CS |

# MOVS	Move String (Byte or Word)

### See Also: MOV, MOVSB, MOVSW, CMPS, LODS, SCAS, STOS, REP, CLD, STD

Flags: not altered

## MOVS destination-string,source-string

Logic:			(ES:DI)  (DS:SI)

if DF = 0

SI  SI + n			; n = 1 for byte, 2 for word scan

DI  DI + n

else

SI  SI - n

DI  DI - n

This instruction copies the byte or word pointed to by DS:SI, into the location pointed to by ES:DI. After the move, SI and DI are incremented (if the direction flag is cleared) or decremented (if the direction flag is set), to point to the next element of the string.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
|  | byte(word) |  |  |  |
| dest,source | 18(26) | 2 | 1 | MOVS WORD_BUFF,INPUT |
| (repeat) dest,source | 9+17(25)/rep | 2/rep | 1 | REP MOVSW |

Note: This instruction is always translated by the assembler into either MOVSB, Move String Byte; or MOVSW, Move String Word, depending upon whether source-string refers to a string of bytes or words. In either case, you must explicitly load the SI and DI registers with the offset of the source and destination strings.

Example: Assuming BUFFER1 as been defined somewhere as:

BUFFFER1DB100 DUP (?)

the following example moves 100 bytes from BUFFER1 to BUFFER2:

CLD						;Move in the forward direction

LEA SI, BUFFER1				;Source address to SI

LEA DI, BUFFER2				;Destination address to DI

MOV CX,100					;CX is used by the REP prefix

REP MOVSBUFFER2, BUFFER1			;...and MOVS it.

# MOVSB	Move String Byte

### See Also: MOV, MOVS, MOVSW, CMPS, LODS, SCAS, STOS, REP, CLD, STD

Flags: not altered

## MOVSB

Logic:			(ES:DI)  (DS:SI)

if DF = 0

SI  SI + 1

DI  DI + 1

else

SI  SI - 1

DI  DI - 1

This instruction copies the byte pointed to by DS:SI into the location pointed to by ES:DI. After the move, SI and DI are incremented (if the direction flag is cleared) or decremented (if the direction flag is set), to point to the next byte.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| - | 18 | 2 | 1 | MOVSB |
| (repeat) | 9+17/rep | 2/rep | 1 | REP MOVSB |

Example: Assuming BUFFER1 as been defined somewhere as:

BUFFFER1DB100 DUP (?)

the following example moves 100 bytes from BUFFER1 to BUFFER2:

CLD					;Move in the forward direction

LEA SI, BUFFER1			;Source address to SI

LEA DI, BUFFER2			;Destination address to DI

MOV CX,100				;CX is used by the REP prefix

REP MOVSB				;...and move it.

# MOVSW	Move String Word

### See also: MOV, MOVS, MOVSB, CMPS, LODS, SCAS, STOS, REP, CLD, STD

Flags: not altered

## MOVSW

Logic:			(ES:DI)  (DS:SI)

if DF = 0

SI  SI + 2

DI  DI + 2

else

SI  SI - 2

DI  DI - 2

This instruction copies the word pointed to by DS:SI to the location pointed to by ES:DI. After the move, SI and DI are incremented (if the direction flag is cleared) or decremented (if the direction flag is set), to point to the next word.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| - | 26 | 2 | 1 | MOVSW |
| (repeat) | 9+25/rep | 2/rep | 1 | REP MOVSW |

Example: Assuming BUFFER1 as been defined somewhere as:

BUFFFER1DB100 DUP (?)

the following example moves 50 words (100 bytes) from BUFFER1 to BUFFER2:

CLD					;Move in the forward direction

LEA SI, BUFFER1			;Source address to SI

LEA DI, BUFFER2			;Destination address to DI

MOV CX,50				;Used by REP; moving 50 words

REP MOVSW				;...and move it.

# MUL	Multiply, Unsigned

### See Also: IMUL, AAM, EA, Flags

| Flags Affected: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | * |  |  |  | ? | ? | ? | ? | * |

## MUL source

Logic:			AX  source * AL		;if source is a byte

or

DX:AX = source * AX		;if source is a word

MUL performs unsigned multiplication. If source is a byte, MUL multiplies source by AL, returning the product in AX. If source is a word, MUL multiplies source by AX, returning the product in DX:AX. The Carry and Overflow flags are set if the upper half of the result (AH for a byte source, DX for a word source) contains any significant digits of the product, otherwise they are cleared.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| reg8 | 70-77 | - | 2 | MUL CH |
| reg16 | 118-133 | - | 2 | MUL BX |
| mem8 | (76-83)+EA | 1 | 2-4 | MUL A_BYTE |
| mem16 | (128-143)+EA | 1 | 2-4 | MUL A_WORD |

# NEG	Negate

### See also: NOT, SUB, SBB, AAS, DAS, EA, Flags

| Flags Affected: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | * |  |  |  | * | * | * | * | * |

## NEG destination

Logic:		destination  -destination		; two's complement

NEG subtracts the destination operand from 0 and returns the result in the destination. This effectively produces the two's complement of the operand. The operand may be a byte or a word.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
|  | byte(word) |  |  |  |
| register | 3 | - | 2 | NEG DL |
| memory | 16(24)+EA | 2 | 2-4 | NEG COEFFICIENT |

Note: If the operand is zero, the carry flag is cleared; in all other cases, the carry flag is set.

Attempting to negate a byte containing -128 or a word containing -32,768 causes no change to the operand and sets the Overflow Flag.

# NOP	No Operation

Flags: not altered

Logic: None

NOP causes the processor to do nothing. This instruction is frequently used for timing purposes, to force memory alignment, and as a "place- holder."

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| no operands | 3 | - | 1 | NOP |

# NOT	Logical NOT

### See Also: NEG, AND, OR, XOR, EA

Flags: not altered

## NOT destination

Logic:			destination  NOT(destination)		; one's complement

NOT inverts each bit of its operand (that is, forms the one's complement). The operand can be a word or byte.

NOT Instruction Logic

| Destination | Result |
| --- | --- |
| 0 | 1 |
| 1 | 0 |

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
|  | byte(word) |  |  |  |
| register | 3 | - | 2 | NOT DX |
| memory | 16(24)+EA | 2 | 2-4 | NOT MASK |

# OR	Logical OR

### See Also: AND, NOT, XOR, EA, Flags

| Flags Affected: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 |  |  |  | * | * | ? | * | 0 |

## OR destination,source

Logic:			destination  destination OR source

OR performs a bit-by-bit logical inclusive OR operation on its operands and returns the result to destination.The operands may be words or bytes.

OR Instruction Logic

| Destination | Source | Result |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 1 |

OR sets each bit of the result to 1 if either or both of the corresponding bits of the operands are 1.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
|  | byte(word) |  |  |  |
| register, register | 3 | - | 2 | OR CH,DL |
| register, memory | 9(13)+EA | 1 | 2-4 | OR BX, MEM_OR_Y |
| memory, register | 16(24)+EA | 2 | 2-4 | OR MEM_OR_Y, BX |
| accumulator, immediate | 4 | - | 2-3 | OR AL,01110110b |
| register, immediate | 4 | - | 3-4 | OR CX,00FFh |
| memory, immediate | 17(25)+EA | 2 | 3-6 | OR MEM_WORD,76h |

# OUT	Output to Port

### See Also: IN

Flags: not altered

## OUT port,accumulator

Logic:			(port)  accumulator

OUT transfers a byte or a word from AL or AX to the specified port. The port may be specified with an immediate byte constant (allowing access to ports 0 through 255) or with a word value in DX (allowing access to ports 0 through 65,535).

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| byte(word) |  |  |  |  |
| immed8, accumulator | 10(14) | 1 | 2 | OUT 254,AX |
| DX, accumulator | 8(12) | 1 | 1 | OUT DX,AL |

Note: It is advised that hardware not use I/O ports F8h through FFh, since these are reserved for controlling the math coprocessor and future processor extensions.

# POP	POP a Word from the Stack

### See Also: PUSH, POPF, MOV, XCHG, XLAT, EA

Flags: not altered

## POP destination

Logic:			destination  (SP)

SP  SP + 2

POP transfers the word at the top of the stack to the destination operand, then increments SP by 2 to point to the new top of stack.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| register | 12 | 1 | 1 | POP CX |
| seg-reg (CS illegal) | 12 | 1 | 1 | POP ES |
| memory | 25+EA | 2 | 2-4 | POP VALUE |

Note: You may not use the CS register as the destination of a POP instruction.

# POPF	POP Flags from the Stack

### See also: POP, PUSH, PUSHF, LAHF, SAHF

| Flags Affected: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | r | r | r | r | r | r | r | r | r |

Logic:			flag-register  (SP)

SP  SP + 2

POPF transfers the word at the top of the stack to the flags register, replacing the old flags, then increments SP by 2 to point to the new top of stack.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| no operands | 12 | 1 | 1 | POPF |

# PUSH	Push Word onto Stack

### See also:POP, POPF, PUSHF, MOV, XCHG, EA

Flags: not altered

## PUSH source

Logic:			SP  SP - 2

(SP)  source

PUSH decrements SP by 2, then copies the operand to the new top of stack. The source of a PUSH instruction cannot be an 8-bit register.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| register | 15 | 1 | 1 | PUSH BX |
| seg-reg (CS illegal) | 14 | 1 | 1 | PUSH ES |
| memory | 24+EA | 2 | 2-4 | PUSH PARAMETERS |

Note: Even if the source refers to a byte in memory, a full word is always pushed.

The 80286 and 80386 microprocessors will push a different value on the stack for the instruction PUSH SP than will the 8086/8088.The 80286 and 80386 push the value of SP before SP is incremented, while the 8086/8088 increments SP first, then pushes SP on the stack.Use the following code instead of a PUSH SP in order to obtain the same results on all microprocessors.

PUSH BP

MOV BP, SP

XCHGBP, [BP]

This code functions in the same manner as a PUSH SP on the 8088/8086.

# PUSHF	Push Flags onto Stack

### See Also: POPF, LAHF, SAHF

Flags: not altered

Logic:			SP  SP - 2

(SP)  flag-register

PUSHF decrements SP by 2, then transfers the flags register to the new top of stack.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| no operands | 14 | 1 | 1 | PUSHF |

# RCL	Rotate through Carry Left

### See Also: ROL, ROR, RCR, SHL, SHR, SAR, SAL, EA, Flags

| Flags Affected: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | * |  |  |  |  |  |  |  | * |

## RCL destination,count

RCL shifts the word or byte at the destination to the left by the number of bit positions specified in the second operand, COUNT. A bit shifted out of the left (high-order) end of the destination enters the carry flag, and the displaced carry flag rotates around to enter the vacated right-most bit position of the destination. This "bit rotation" continues the number of times specified in COUNT. (Another way of looking at this is to consider the carry flag as the highest order bit of the word being rotated.)

If COUNT is not equal to 1, the Overflow flag is undefined. If COUNT is equal to 1, then the Overflow Flag is set to the XOR of the top 2 bits of the original operand.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
|  | byte(word) |  |  |  |
| register, 1 | 2 | - | 2 | RCL CX,1 |
| register, CL | 8+4/bit | - | 2 | RCL BL,CL |
| memory, 1 | 15(23)+EA | 2 | 2-4 | RCL MULTIPLY_X_2,1 |
| memory, CL | 20(28)+EA+4/bit | 2 | 2-4 | RCL MOVE_AROUND,CL |

Note: COUNT is normally taken as the value in CL. If, however, you wish to rotate only one position, replace the second operand, CL, with the value 1, as shown in the first example above.

The 80286 and 80386 microprocessors limit the COUNT value to 31.If the COUNT is greater than 31, these microprocessors use COUNT MOD 32 to produce a new COUNT between 0 and 31.This upper bound exists to limit the amount of time that an interrupt response will be delayed waiting for the instruction to complete.

Multiple RCLs that use 1 as the COUNT may be faster and require less memory than a single RCL that uses CL for COUNT.

The overflow flag is undefined if the rotate count is greater than 1.

# RCR	Rotate through Carry Right

### See Also: ROR, RCL, ROL, SAR, SHR, SHL, SAL, EA, Flags

| Flags Affected: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | * |  |  |  |  |  |  |  | * |

## RCR destination, count

RCR shifts the word or byte at the destination to the right by the number of bit positions specified in the second operand, COUNT. A bit shifted out of the right (low-order) end of the destination enters the carry flag, and the displaced carry flag rotates around to enter the vacated left-most bit position of the destination. This "bit rotation" continues the number of times specified in COUNT. (Another way of looking at this is to consider the carry flag as the lowest order bit of the word being rotated.)

If COUNT is not equal to 1, the Overflow flag is undefined. If COUNT is equal to 1, the Overflow Flag is set to the XOR of the top 2 bits of the result.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
|  | byte(word) |  |  |  |
| register, 1 | 2 | - | 1 | RCR CX,1 |
| register, CL | 8+4/bit | - | 2 | RCR DL,CL |
| memory, 1 | 15(23)+EA | 2 | 2-4 | RCR DIVIDE_BY_2,1 |
| memory, CL | 20(28)+EA+4/bit | 2 | 2-4 | RCR AROUND_MOVE,CL |

Note: COUNT is normally taken as the value in CL. If, however, you wish to rotate only one position, replace the second operand, CL, with the value 1, as shown in the first example above.

The 80286 and 80386 microprocessors limit the COUNT value to 31.If COUNT is greater than 31, these microprocessors use COUNT MOD 32 to produce a new COUNT between 0 and 31.This upper bound exists to limit the amount of time an interrupt response will be delayed waiting for the instruction to complete.

Multiple RCRs that use 1 as the COUNT may be faster and require less memory than a single RCR that uses CL for COUNT.

The overflow flag is undefined if the rotate count is greater than 1.

# REP	Repeat

### See Also: REPNE, MOVS, STOS, CMPS, SCAS, LODS, CLD, STD

Flags: not altered

## REP string-instruction

Logic:			while CX <> 0				;for MOVS, LODS or STOS

execute string instruction

CX  CX - 1

while CX <> 0

execute string instruction		;for CMPS or SCAS

CX  CX - 1

if ZF = 0 terminate loop

REP is a prefix that may be specified before any string instruction (CMPS, LODS, MOVS, SCAS, and STOS). REP causes the string instruction following it to be repeated, as long as CX does not equal 0; CX is decremented after each execution of the string instruction.(For CMPS and SCAS, REP will also terminate the loop if the Zero Flag is clear after the string instruction executes.)

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| - | 2 | - | 1 | REP MOVS TO,FROM |

Note: If CX is initially 0, the REPeated instruction is skipped entirely.

The test for CX equal to 0 is performed before the instruction is executed.The test for the Zero Flag clear--done only for CMPS and SCAS--is performed after the instruction is executed.

REP, REPE (Repeat While Equal), and REPZ (Repeat While Zero) are all synonyms for the same instruction.

REPNZ (Repeat Not Zero) is similar to REP, but when used with CMPS and SCAS, will terminate with the Zero Flag set, rather than cleared.

REP is generally used with the MOVS (Move String) and STOS (Store String) instructions; it can be thought of as "repeat while not end of string."

You do not need to initialize ZF before using repeated string instructions.

A REPeated instruction that is interrupted between repeats will correctly resume processing upon return from the interrupt. However, if other prefixes are used on a single instruction (for example, segment override or LOCK) in addition to the REP, all prefixes except the one that immediately preceded the string instruction will be lost. Therefore, if you must use more than one prefix on a single instruction, you should disable interrupts before the instruction, and enable them afterwards. Note that even this precaution will not prevent a non-maskable interrupt, and that lengthy string operations may cause large delays in interrupt processing.

Example: The following example moves 100 bytes from BUFFER1 to BUFFER2:

CLD					;Move in the forward direction

LEA SI, BUFFER1			;Source pointer to SI

LEA DI, BUFFER2			;...and destination to DI

MOV CX,100				;REP uses CX as the counter

REP MOVSB				;...and do it

# REPE	Repeat While Equal

### See Also: REP

Flags: not altered

This instruction is a synonym for REP. See the description for REP.

# REPNE	Repeat While not Equal

### See Also: REP, MOVS, STOS, CMPS, SCAS, LODS, CLD, STD

Flags: not altered

## REPNE string-instruction

Logic:			while CX <> 0					;for MOVS, LODS or STOS

execute string instruction

CX  CX - 1

while CX <> 0					;for CMPS or SCAS

execute string instruction

CX  CX - 1

if ZF <> 0 terminate loop			;This is only difference

;between REP and REPNE

REPNE is a prefix that may be specified before any string instruction (CMPS, LODS, MOVS, SCAS, and STOS). REPNE causes the string instruction following it to be repeated, as long as CX does not equal 0; CX is decremented after each execution of the string instruction. (For CMPS and SCAS, REP will also terminate the loop if the Zero Flag is set after the string instruction executes. Compare this to REP, which will terminate if the Zero Flag is clear.)

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| - | 2 | - | 1 | REPNE SCASB |

Note:If CX is initially 0, the REPeated instruction is skipped entirely.

The test for CX equal to 0 is performed before the instruction is executed.The test for the Zero Flag set--done only for CMPS and SCAS--is performed after the instruction is executed.

REPNE and REPNZ are synonyms for the same instruction.

You do not need to initialize ZF before using repeated string instructions.

A repeated instruction that is interrupted between repeats will correctly resume processing upon return from the interrupt. However, if other prefixes are used on a single instruction (for example, segment override or LOCK) in addition to the REP, all prefixes except the one that immediately preceded the string instruction will be lost. Therefore, if you must use more than one prefix on a single instruction, you should disable interrupts before the instruction, and enable them afterwards. Note that even this precaution will not prevent a non-maskable interrupt, and that lengthy string operations may cause large delays in interrupt processing.

Example: The following example will find the first byte equal to 'A' in the 100-byte buffer at STRING.

CLD					;Scan string in forward direction

MOV AL,'A'				;Scan for 'A'

LEA DI, STRING			;Address to start scanning at

MOV CX,100				;Scanning 100 bytes

REPNE SCASB				; ...and scan it

DEC DI					;Back up DI to point to the 'A'

Upon termination of the repeated SCASB instruction, CX will be equal to zero if a byte value of 'A' was not found in STRING, and non-zero if it was.

# REPNZ	Repeat While Not Zero

### See Also: REPNE

Flags: not altered

REPNZ is the same instruction as REPNE. See the description for REPNE.

# REPZ	Repeat While Zero

### See Also: REP

Flags: not altered

This instruction is a synonym for REP. See the description for REP.

# RET	Return from Procedure

### See Also: IRET, CALL, JMP

Flags: not altered

## RET optional-pop-value

Logic:			POP IP

If FAR RETURN (inter-segment)

POP CS

SP  SP + optional-pop-value (if specified)

RET transfers control from a called procedure back to the instruction following the CALL, by:

- Popping the word at the top of the stack into IP

- If the return is an intersegment return:

- Popping the word now at the top of the stack into CS

- Adding the optional-pop-value, if specified, to SP

The assembler will generate an intrasegment RET if the procedure containing the RET was designated by the programmer as NEAR, and an intersegment RET if it was designated FAR. The optional-pop-value specifies a value to be added to SP, which has the effect of popping the specified number of bytes from the top of the stack.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| (intrasegment, no pop) | 20 | 1 | 1 | RET |
| (intrasegment, with pop) | 24 | 1 | 3 | RET 4 |
| (intersegment, no pop) | 32 | 2 | 1 | RET |
| (intersegment, pop) | 31 | 2 | 3 | RET 2 |

# ROL	Rotate Left

### See Also: RCL, ROR, RCR, SAL, SAR, SHL, SHR, EA, Flags

| Flags Affected: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | * |  |  |  |  |  |  |  | * |

## ROL destination,count

ROL shifts the word or byte at the destination to the left by the number of bit positions specified in the second operand, COUNT. As bits are transferred out the left (high-order) end of the destination, they re-enter on the right (low-order) end. The Carry Flag is updated to match the last bit shifted out of the left end.

If COUNT is not equal to 1, the Overflow flag is undefined. If COUNT is equal to 1, then the Overflow Flag is set to the XOR of the top 2 bits of the original operand.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| byte(word) |  |  |  |  |
| register, 1 | 2 | - | 2 | ROL DI,1 |
| register, CL | 8+4/bit | - | 2 | ROL BX,CL |
| memory, 1 | 15(23)+EA | 2 | 2-4 | ROL BYTE,1 |
| memory, CL | 20(28)+EA+4/bit | 2 | 2-4 | ROL WORD,CL |

Note: COUNT is normally taken as the value in CL. If, however, you wish to rotate only one position, replace the second operand, CL, with the value 1, as shown in the first example above.

The 80286 and 80386 microprocessors limit the COUNT value to 31.If COUNT is greater than 31, these microprocessors will use COUNT MOD 32 to produce a new COUNT between 0 and 31.This upper boundary exists to limit the amount of time that an interrupt response will be delayed waiting for the instruction to complete.

Multiple ROLs that use 1 as the COUNT may be faster and require less memory than a single ROL that uses CL for COUNT.

The overflow flag is undefined when the rotate count is greater than 1.

# ROR	Rotate Right

### See Also: RCR, ROL, RCL, SAR, SAL, SHL, SHR, EA, Flags

| Flags Affected: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | * |  |  |  |  |  |  |  | * |

## ROR destination,count

ROR shifts the word or byte at the destination to the right by the number of bit positions specified in the second operand, COUNT. As bits are transferred out the right (low-order) end of the destination, they re-enter on the left (high-order) end. The Carry Flag is updated to match the last bit shifted out of the right end.

If COUNT is not equal to 1, the Overflow flag is undefined. If COUNT is equal to 1, the Overflow Flag is set to the XOR of the top 2 bits of the result.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
|  | byte(word) |  |  |  |
| register, 1 | 2 | - | 2 | ROR BL,1 |
| register, CL | 8+4/bit | - | 2 | ROR AX,CL |
| memory, 1 | 15(23)+EA | 2 | 2-4 | ROR WORD,1 |
| memory, CL | 20(28)+EA+4/bit | 2 | 2-4 | ROR BYTE,CL |

Note: COUNT is normally taken as the value in CL. If, however, you wish to rotate by only one position, replace the second operand, CL, with the value 1, as shown in the first example above.

The 80286 and 80386 microprocessors limit the COUNT value to 31.If the COUNT is greater than 31, these microprocessors use COUNT MOD 32 to produce a new COUNT between 0 and 31.This upper bound exists to limit the amount of time an interrupt response will be delayed waiting for the instruction to complete.

Multiple RORs that use 1 as the COUNT may be faster and require less memory than a single ROR that uses CL for COUNT.

The overflow flag is undefined when the rotate count is greater than 1.

# SAHF	Store Register AH into Flags

### See Also: LAHF, PUSHF, POPF, Flags

| Flags Affected: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | * | * | * | * | * |

Logic:			Flag-reg bits  AH bits

S Z A P C 7 6 4 2 0

SAHF copies bits 7, 6, 4, 2, and 0 from the AH register into the Flags register, replacing the previous values of the Sign, Zero, Auxiliary Carry, Parity, and the Carry flags.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| no operands | 4 | - | 1 | SAHF |

Note: The Overflow, Direction, Interrupt, and Trap flags are not changed by this instruction.  This instruction is primarily used to provide upward compatibility between the 8080/8085 family and the 8086 family.

# SAL	Shift Arithmetic Left

### See also: SHL, SAR, SHR, RCL, RCR, ROL, ROR, EA, Flags

| Flags Affected: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | * |  |  |  | * | * | ? | * | * |

## SAL destination,count

SAL shifts the word or byte at the destination to the left by the number of bit positions specified in the second operand, COUNT. As bits are transferred out the left (high-order) end of the destination, zeroes are shifted in the right (low-order) end. The Carry Flag is updated to match the last bit shifted out of the left end.  If COUNT is not equal to 1, the Overflow flag is undefined. If COUNT is equal to 1, the Overflow Flag is cleared if the top 2 bits of the original operand were the same, otherwise the Overflow Flag is set.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
|  | byte(word) |  |  |  |
| register, 1 | 2 | - | 2 | SAL AL,1 |
| register, CL | 8+4/bit | - | 2 | SAL SI,CL |
| memory, 1 | 15(23)+EA | 2 | 2-4 | SAL WORD,1 |
| memory, CL | 20(28)+EA+4/bit | 2 | 2-4 | SAL BYTE,CL |

Note: COUNT is normally taken as the value in CL. If, however, you wish to shift only one position, replace the second operand, CL, with the value 1, as shown in the first example above.

The 80286 and 80386 microprocessors limit the COUNT value to 31.If the COUNT is greater than 31, these microprocessors use COUNT MOD 32 to produce a new COUNT between 0 and 31.This upper bound exists to limit the amount of time an interrupt response will be delayed waiting for the instruction to complete.

Multiple SALs that use 1 as the COUNT may be faster and require less memory than a single SAL that uses CL for COUNT.

SHL, Shift Logical Left, is the same instruction; SHL is a synonym for SAL.

The overflow flag is undefined when the shift count is greater than 1.

# SAR	Shift Arithmetic Right

### See Also: SHR, SAL, SHL, RCR, RCL, ROR, ROL, EA, Flags

| Flags Affected: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | * |  |  |  | * | * | ? | * | * |

## SAR destination,count

SAR shifts the word or byte in destination to the right by the number of bit positions specified in the second operand, COUNT. As bits are transferred out the right (low-order) end of the destination, bits equal to the original sign bit are shifted into the left (high-order) end, thereby preserving the sign bit. The Carry Flag is set equal to the last bit shifted out of the right end.  If COUNT is not equal to 1, the Overflow flag is undefined. If COUNT is equal to 1, the Overflow flag is cleared.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
|  | byte(word) |  |  |  |
| register, 1 | 2 | - | 2 | SAR DX,1 |
| register, CL | 8+4/bit | - | 2 | SAR DI,CL |
| memory, 1 | 15(23)+EA | 2 | 2-4 | SAR N_BLOCKS,1 |
| memory, CL | 20(28)+EA+4/bit | 2 | 2-4 | SAR N_BLOCKS,CL |

Note: COUNT is normally taken as the value in CL. If, however, you wish to shift only one position, replace the second operand, CL, with the value 1, as shown in the first example above.

The 80286 and 80386 microprocessors limit the COUNT value to 31.If the COUNT is greater than 31, these microprocessors use COUNT MOD 32 to produce a new COUNT between 0 and 31.This upper bound exists to limit the amount of time an interrupt response will be delayed waiting for the instruction to complete.

Multiple SARs that use 1 as the COUNT may be faster and require less memory than a single SAR that uses CL for COUNT.

The overflow flag is undefined when the shift count is greater than 1.

# SBB	Subtract with Borrow

### See Also: SUB, DEC, NEG, AAS, DAS, EA, Flags

| Flags Affected: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | * |  |  |  | * | * | * | * | * |

## SBB destination,source

Logic:			destination  destination - source - CF

SBB subtracts the source from the destination, subtracts 1 from that result if the Carry Flag is set, and stores the result in destination. The operands may be bytes or words, and both may be signed or unsigned binary numbers.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
|  | byte(word) |  |  |  |
| register, register | 3 | - | 2 | SBB DX,AX |
| register, memory | 9(13)+EA | 1 | 2-4 | SBB DX,FEE |
| memory, register | 16(24)+EA | 2 | 2-4 | SBB SIGH,SI |
| accumulator, immediate | 4 | - | 2-3 | SBB AX,8 |
| register, immediate | 4 | - | 3-4 | SBB BH,4 |
| memory, immediate | 17(25)+EA | 2 | 3-6 | SBB TOTAL,10 |

Note: SBB is useful for subtracting numbers that are larger than 16 bits, since it subtracts a borrow (in the carry flag) from a previous operation.

You may subtract a byte-length immediate value from a destination which is a word; in this case, the byte is sign-extended to 16 bits before the subtraction.

# SCAS	Scan String (Byte or Word)

### See Also: SCASB, SCASW, CMP, CMPS, REP, CLD, STD, Flags

| Flags Affected: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | * |  |  |  | * | * | * | * | * |

## SCAS destination-string

Logic:				CMP Accumulator, (ES:DI);Sets flags only

if DF = 0

DI  DI + n ;n = 1 for byte, 2 for word

else

DI  DI - n

This instruction compares the accumulator (AL or AX) to the byte or word pointed to by ES:DI. SCAS sets the flags according to the results of the comparison; the operands themselves are not altered. After the comparison, DI is incremented (if the direction flag is cleared) or decremented (if the direction flag is set), in preparation for comparing the next element of the string.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
|  | byte(word) |  |  |  |
| dest-str | 15(19) | 1 | 1 | SCAS WORD_TABLE |
| (repeat) dest-str | 9+15(19)/rep | 1/rep | 1 | REPNE SCAS BYTE_TABLE |

Note: This instruction is always translated by the assembler into either SCASB, Scan String Byte, or SCASW, Scan String Word, depending upon whether destination-string refers to a string of bytes or words. In either case, however, you must explicitly load the DI register with the offset of the string.

SCAS is useful for searching a block for a given byte or word value. Use CMPS, Compare String, if you wish to compare two strings (or blocks) in memory, element by element.

Example: Assuming the definition:

LOST_A DB 100DUP(?)

the following example searches the 100-byte block starting at LOST_A for the character 'A' (65 decimal).

MOV AX, DS

MOV ES, AX				;SCAS uses ES:DI, so copy DS to ES

CLD					;Scan in the forward direction

MOV AL, 'A'				;Searching for the lost 'A'

MOV CX,100				;Scanning 100 bytes (CX is used by REPNE)

LEA DI, LOST_A			;Starting address to DI

REPNE SCASLOST_A			;...and scan it.

JE FOUND				;The Zero Flag will be set if we found a match.

NOTFOUND:			.		;If we get here, no match was found

.

.

FOUND:	DEC DI				;Back up DI so it points to the first

.		; matching 'A'

.

Upon exit from the REPNE SCAS loop, the Zero Flag will be set if a match was found, and cleared otherwise. If a match was found, DI will be pointing one byte past the match location; the DEC DI at FOUND takes care of this problem.

# SCASB	Scan String Byte

### See Also: SCAS, SCASW, CMPS, REP, CLD, STD, Flags

| Flags Affected: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | * |  |  |  | * | * | * | * | * |

## SCASB

Logic:			CMP AL, (ES:DI)	; Sets flags only

if DF = 0

DI  DI + 1

else

DI  DI - 1

This instruction compares two bytes by subtracting the destination byte, pointed to by ES:DI, from AL. SCASB sets the flags according to the results of the comparison. The operands themselves are not altered. After the comparison, DI is incremented (if the direction flag is cleared) or decremented (if the direction flag is set), in preparation for comparing the next byte.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| - | 15 | 1 | 1 | SCASB |
| (repeat) | 9+15/rep | 1/rep | 1 | REPNE SCASB |

Note: SCAS is useful for searching a block for a given byte or word value. Use CMPS, Compare String, if you wish to compare two strings (or blocks) in memory, element by element.

Example: The following example searches the 100-byte block starting at LOST_A for the character 'A' (65 decimal).

MOV AX, DS

MOV ES, AX				;SCAS uses ES:DI, so copy DS to ES

CLD					;Scan in the forward direction

MOV AL, 'A'				;Searching for the lost 'A'

MOV CX,100				;Scanning 100 bytes (CX is used by REPNE)

LEA DI, LOST_A			;Starting address to DI

REPNE SCASB				; ...and scan it.

JE FOUND				;The Zero Flag will be set if we found a match.

NOTFOUND:			.		;If we get here, no match was found

.

.

FOUND:	DEC DI				;Back up DI so it points to the first

.		;matching 'A'

.

Upon exit from the REPNE SCASB loop, the Zero Flag will be set if a match was found, and cleared otherwise. If a match was found, DI will be pointing one byte past the match location; the DEC DI at FOUND takes care of this problem.

# SCASW	Scan String Word

### See Also: SCAS, SCASB, CMPS, REP, CLD, STD, Flags

| Flags Affected: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | * |  |  |  | * | * | * | * | * |

## SCASW

Logic:			CMP AX, (ES:DI) ; Sets flags only

if DF = 0

DI  DI + 2

else

DI  DI - 2

This instruction compares two words by subtracting the destination word, pointed to by ES:DI, from AX. SCASW sets the flags according to the results of the comparison. The operands themselves are not altered. After the comparison, DI is incremented (if the direction flag is cleared) or decremented (if the direction flag is set), in preparation for comparing the next word.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| - | 19 | 1 | 1 | SCASW |
| (repeat) | 9+19/rep | 1/rep | 1 | REPNE SCASW |

Note: SCAS is useful for searching a block for a given byte or word value. Use CMPS, Compare String, if you wish to compare two strings (or blocks) in memory, element by element.

Example: The following example searches the 100-byte block starting at LOST_A for the character 'A' (65 decimal).

MOV AX, DS

MOV ES, AX				;SCAS uses ES:DI, so copy DS to ES

CLD					;Scan in the forward direction

MOV AL, 'A'				;Searching for the lost 'A'

MOV CX,50				;Scanning 50 words (CX is used by REPNE)

LEA DI, LOST_A			;Starting address to DI

REPNE SCASW				; ...and scan it.

JE FOUND				;The Zero Flag will be set if we found a match.

REPNE SCASW				; ...and scan it.

JE FOUND				;The Zero Flag will be set if we found a match.

NOTFOUND:			.		;If we get here, no match was found

.

.

FOUND: DEC DI				;Back up DI so it points to the first

DEC DI					;matching 'A'

.

.

Upon exit from the REPNE SCASW loop, the Zero Flag will be set if a match was found, and cleared otherwise. If a match was found, DI will be pointing two bytes (one word) past the match location; the DEC DI pair at FOUND takes care of this problem.

# SHL	Shift Logical Left

### See Also: SAL, SHR, SAR, RCL, RCR, ROL, ROR, EA, Flags

| Flags Affected: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | * |  |  |  | * | * | ? | * | * |

## SHL destination,count

SHL is the same instruction as SAL, Shift Arithmetic Left. SHL shifts the word or byte at the destination to the left by the number of bit positions specified in the second operand, COUNT. As bits are transferred out the left (high-order) end of the destination, zeroes are shifted in the right (low-order) end. The Carry Flag is updated to match the last bit shifted out of the left end.

If COUNT is not equal to 1, the Overflow flag is undefined. If COUNT is equal to 1, the Overflow Flag is cleared if the top 2 bits of the original operand were the same, otherwise the Overflow Flag is set.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
|  | byte(word) |  |  |  |
| register, 1 | 2 | - | 2 | SHL AL,1 |
| register, CL | 8+4/bit | - | 2 | SHL SI,CL |
| memory, 1 | 15(23)+EA | 2 | 2-4 | SHL WORD,1 |
| memory, CL | 20(28)+EA+4/bit | 2 | 2-4 | SHL BYTE,CL |

Note: COUNT is normally taken as the value in CL. If, however, you wish to shift only one position, replace the second operand, CL, with the value 1, as shown in the first example below.

The 80286 and 80386 microprocessors limit the COUNT value to 31.If the COUNT is greater than 31, these microprocessors use COUNT MOD 32 to produce a new COUNT between 0 and 31.This upper bound exists to limit the amount of time an interrupt response will be delayed waiting for the instruction to complete.

Multiple SHLs that use 1 as the COUNT may be faster and require less memory than a single SHL that uses CL for COUNT.

The overflow flag is undefined when the shift count is greater than 1.

# SHR	Shift Logical Right

### See Also: SAR, SHL, SAL, RCR, RCL, ROR, ROL, EA, Flags

| Flags Affected: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | * |  |  |  | * | * | ? | * | * |

## SHR destination,count

SAR shifts the bits in destination to the right by the number of positions specified in the count operand (or in CL, if no count operand is included). 0s are shifted in on the left. If the sign bit retains its original value, the Overflow Flag is cleared; it is set if the sign changes. The Carry Flag is updated to reflect the last bit shifted.

If COUNT is not equal to 1, the Overflow flag is undefined, otherwise the Overflow Flag is set to the high-order bit of the original operand.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| register, 1 | 2 | - | 2 | SHR SI,1 |
| register, CL | 8+4/bit | - | 2 | SHR SI,CL |
| memory, 1 | 15+EA | 2 | 2-4 | SHR ID_BYTE[SI][BX], |
| memory, CL | 20+EA+4/bit | 2 | 2-4 | SHR INPUT_WORD,CL |

Note: COUNT is normally taken as the value in CL. If, however, you wish to shift only one position, replace the second operand, CL, with the value 1, as shown in the first example below.

The 80286 and 80386 microprocessors limit the COUNT value to 31.If the COUNT is greater than 31, these microprocessors use COUNT MOD 32 to produce a new COUNT between 0 and 31.This upper bound exists to limit the amount of time an interrupt response will be delayed waiting for the instruction to complete.

Multiple SHRs that use 1 as the COUNT may be faster and require less memory than a single SHR that uses CL for COUNT.

The overflow flag is undefined when the shift count is greater than 1.

# STC	Set Carry Flag

### See Also: CLC, CMC, STD, CLD, STI, CLI, Flags

| Flags Affected: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | 1 |

Logic:			CF  1

STC sets the Carry Flag. No other flags are affected.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| no operands | 2 | - | 1 | STC |

# STD	Set Direction Flag

### See Also: CLD, STC, CLC, CMC, STI, CLI, Flags

| Flags Affected: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 |  |  |  |  |  |  |  |

Logic:			DF  1 (Decrement in string instructions)

STD sets the Direction Flag. No other flags are affected. Setting the direction flag causes string operations to decrement SI and DI.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| no operands | 2 | - | 1 | STD |

# STI	Set Interrupt Enable Flag

### See Also: CLI, STC, CLC, CMC, STD, CLD, Flags

| Flags Affected: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 |  |  |  |  |  |  |

Logic:			IF  1

STI sets the Interrupt Enable Flag, permitting the processor to recognize maskable interrupts. No other flags are affected. (Non- maskable interrupts are recognized no matter what the state of the interrupt enable flag.)

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| no operands | 2 | - | 1 | STI |

Note: A pending interrupt will not be recognized until after the instruction following the STI executes.

# STOS	Store String (Byte or Word)

### See Also: STOSB, STOSW, CMPS, LODS, MOVS, SCAS, REP, CLD, STD

Flags: not altered

## STOS destination-string

Logic:			(ES:DI)  Accumulator

if DF = 0

DI  DI + n		; n = 1 for byte, 2 for word scan

else

DI  DI - n

STOS copies the value (byte or word) in AL or AX into the location pointed to by ES:DI. DI is then incremented (if the direction flag is cleared) or decremented (if the direction flag is set), in preparation for storing the accumulator in the next location.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
|  | byte(word) |  |  |  |
| dest-string | 11(15) | 1 | 1 | STOS WORD_ARRAY |
| (repeat) dest-string | 9+10(14)/rep | 1/rep | 1 | REP STOS BYTE_ARRAY |

Note: This instruction is always translated by the assembler into either STOSB, Store String Byte, or STOSW, Store String Word, depending upon whether destination-string refers to a string of bytes or words. In either case, however, you must explicitly load the DI register with the offset of the string.

Example: When used in conjunction with the REP prefixes, the Store String instructions are useful for initializing a block of memory. For example, the following code would initialize the 100-byte memory block at BUFFER to 0:

MOV AL,0				;The value to initialize BUFFER to

LEA DI,BUFFER			;Starting location of BUFFER

MOV CX,100				;Size of BUFFER

CLD					;Let's move in forward direction

REP STOS BUFFER			;Compare this line to example for STOSB

# STOSB	Store String Byte

### See Also: STOS, STOSW, CMPS, LODS, MOVS, SCAS, REP, CLD, STD

Flags: not altered

## STOSB

Logic:			(ES:DI)  AL

if DF = 0

DI  DI + 1

else

DI  DI - 1

STOSB copies the value in AL into the location pointed to by ES:DI. DI is then incremented (if the direction flag is cleared) or decremented (if the direction flag is set), in preparation for storing AL in the next location.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| - | 11 | 1 | 1 | STOSB |
| (repeat) | 9+10/rep | 1/rep | 1 | REP STOSB |

When used in conjunction with the REP prefixes, the Store String instructions are useful for initializing a block of memory. For example, the following code would initialize the 100-byte memory block at BUFFER to 0:

MOV AL,0				;The value to initialize BUFFER to

LEA DI,BUFFER			;Starting location of BUFFER

MOV CX,100				;Size of BUFFER

CLD					;Let's move in forward direction

REP STOSB				;Compare this line to example for STOS

# STOSW	Store String Word

### See Also: STOS, STOSB, CMPS, LODS, MOVS, SCAS, REP, CLD, STD

Flags: not altered

Logic:			(ES:DI)  AX

if DF = 0

DI  DI + 2

else

DI  DI - 2

STOSW copies the value in AX into the location pointed to by ES:DI. DI is then incremented (if the direction flag is cleared) or decremented (if the direction flag is set), in preparation for storing AX in the next location.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| - | 15 | 1 | 1 | STOSW |
| (repeat) | 9+14/rep | 1/rep | 1 | REP STOSW |

When used in conjunction with the REP prefixes, the Store String instructions are useful for initializing a block of memory. For example, the following code would initialize the 100-byte memory block at BUFFER to 0:

MOV AX,0				;The value to initialize BUFFER to

LEA DI,BUFFER			;Starting location of BUFFER

MOV CX,50				;Size of BUFFER, in words

CLD					;Let's move in forward direction

REP STOSW				;Compare this line to example for STOS

# SUB	Subtract

### See Also: SBB, DEC, NEG, CMP, AAS, DAS, EA, Flags

| Flags Affected: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | * |  |  |  | * | * | * | * | * |

## SUB destination,source

Logic:			destination  destination - source

SUB subtracts the source operand from the destination operand and stores the result in destination. Both operands may be bytes or words, and both may signed or unsigned binary numbers.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
|  | byte(word) |  |  |  |
| register, register | 3 | - | 2 | SUB DX,BX |
| register, memory | 9(13)+EA | 1 | 2-4 | SUB DX,TOTAL |
| memory, register | 16(24)+EA | 2 | 2-4 | SUB RATE,CL |
| accumulator, immediate | 4 | - | 2-3 | SUB AH,25 |
| register, immediate | 4 | - | 3-4 | SUB DX,5280 |
| memory, immediate | 17(25)+EA | 2 | 3-6 | SUB RESULT,1032 |

Note: You may wish to use SBB if you need to subtract numbers that are larger than 16 bits, since SBB subtracts a borrow from a previous operation.

You may subtract a byte-length immediate value from a destination which is a word; in this case, the byte is sign-extended to 16 bits before the subtraction.

# TEST	Test

### See Also: CMP, AND, NOT, OR, XOR, EA, Flags

| Flags Affected: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 |  |  |  | * | * | ? | * | 0 |

## TEST destination,source

Logic:			(destination AND source)		; Set flags only

CF  0

OF  0

TEST performs a logical AND on its two operands and updates the flags. Neither the destination nor source is changed.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
|  | byte(word) |  |  |  |
| register, register | 3 | - | 2 | TEST SI,DX |
| register, memory | 9(13)+EA | 1 | 2-4 | TEST SI,MASK |
| accumulator, immediate | 4 | - | 2-3 | TEST AL,00000100b |
| register, immediate | 5 | - | 3-4 | TEST CX,1234 |
| memory, immediate | 11+EA | - | 3-6 | TEST PARAM,1F1Fh |

TEST is useful for examining the status of individual bits. For example, the following section of code will transfer control to ONE_FIVE_OFF if both bits one and five of register AL are cleared. The status of all other bits will be ignored.

TEST AL,00100010b			;Mask out all bits except 1 and 5

JZ	ONE_FIVE_OFF			;If either was set, result was not 0

NOT_BOTH:			.		;One or both bits was set

ONE_FIVE_OFF:				;Bits 1 and 5 were off

.

.

# WAIT	Wait

### See Also: HLT, ESC, LOCK

Flags: not altered

Logic:		None

WAIT causes the processor to enter a wait state. The processor will remain inactive until the TEST input on the microprocessor is driven active.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| no operands | 3+5n | - | 1 | WAIT |

Note: This instruction is used to synchronize external hardware, such as a coprocessor.

# XCHG	Exchange Registers

### See Also: MOV, PUSH, POP, XLAT, EA

Flags: not altered

## XCHG destination,source

Logic:		destination -> source

XCHG switches the contents of its operands, which may be either bytes or words.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
|  | byte(word) |  |  |  |
| accumulator, reg16 | 3 | - | 1 | XCHG AX,SI |
| memory, register | 17(25)+EA | 2 | 2-4 | LOCK XCHG SEMPHOR, DX |
| register, register | 4 | - | 2 | XCHG CL,DL |

Note: Used in conjunction with the LOCK prefix, this instruction is particularly useful when implementing semaphores to control shared resources.

# XLAT	Translate

Flags: not altered

## XLAT translate-table

Logic:			AL  (BX + AL)

XLAT translates bytes via a table lookup. A pointer to a 256-byte translation table is loaded into BX. The byte to be translated is loaded into AL; it serves as an index (ie, offset) into the translation table. After the XLAT instruction is executed, the byte in AL is replaced by the byte located AL bytes from the beginning of the translate-table.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| translate-table | 11 | 1 | 1 | XLAT SINE_TABLE |

Note: Translate-table can be less than 256 bytes.

The operand, translate-table, is optional since a pointer to the table must be loaded into BX before the instruction is executed.

The following example translates a decimal value (0 to 15) to the corresponding single hexadecimal digit.

lea bx, hex_table			;pointer to table into BX

mov al, decimal_digit		;digit to be translated to AL

xlat hex_table			;translate the value in AL

;AL now contains ASCII hex digit

.

hex_table db '0123456789ABCDEF'

# XOR	Exclusive OR

### See also:OR, AND, NOT, EA, Flags

| Flags Affected: | O | D | I | T | S | Z | A | P | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 |  |  |  | * | * | * | * | 0 |

## XOR destination,source

Logic:			destination  destination XOR source

XOR performs a bit-by-bit "exclusive or" on its two operands, and returns the result in the destination operand.The operands may be bytes or words.

XOR Instruction Logic

| Destination | Source | Result |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 0 |

XOR sets each bit of the result to 1 only one of the corresponding bits is set to one.

| Operands | Clocks | Transfers | Bytes | Example |
| --- | --- | --- | --- | --- |
| register, register | 3 | - | 2 | XOR CX,BX |
| register, memory | 9(13)+EA | 1 | 2-4 | XOR CL,MASK_BYTE |
| memory, register | 16(24)+EA | 2 | 2-4 | XOR ALPHA[SI],DX |
| accumulator, immediate | 4 | - | 2-3 | XOR AL,01000001b |
| register, immediate | 4 | - | 3-4 | XOR SI,00C2h |
| memory, immediate | 17(25)+EA | 2 | 3-6 | XOR RETURN_CODE,0D2h |

FIM